Wave Energy Prize

RTI Wave Power Test Plan

August, 1 2016

This page intentionally left blank

# Content

Content	3

List of Tables	5

List of Figures	6

List of Abbreviations	8

List of Notations	9

1	Introduction	11

2	Test Objective	12

3	Test Facility	13

3.1	Wave Maker	15

3.2	MASK Orientation	16

4	Scaled Model Description	18

4.1	Device description	18

4.2	Power Take-Off description	20

4.3	Device properties	22

4.4	Froude scaling	29

4.5	Other scaling or measures	30

4.6	Control strategy	31

5	Test Matrix and Schedule	33

5.1	Test matrix	33

5.2	Test schedule	33

6	Experimental Set Up and Methods	36

6.1	Mooring	36

6.2	Instrumentation	39

6.3	Motion tracking	48

6.4	RTI F2 QD Deployment and Retrieval Summary	50

7	Data Processing and Analysis	55

7.1	Data quality assurance and on-site processing	55

7.2	Data analysis	55

7.2.1	“Real Time” Data QA	57

7.2.2	Settling Interval and Time between Test Data QA	57

7.3	Uncertainty Analysis	59

8	Data Management	60

Appendix A: Device electrical and mechanical drawings	61

Appendix B: PTO calibration results	66

Appendix C: Device Froude scaling	67

Appendix D: Detailed description of control strategy	69

Appendix E: Wave environment calibration	71

Appendix F: Raw Data Channel list	72

Appendix G: Checklists for spot checks, software operation, readiness verification, and “real time” data QA	73

Spot Check Procedure	74

Appendix H: Team provided sensors – specifications and calibrations	77

Appendix I: Wave Energy Prize provided sensors – specifications and calibrations	86

Appendix J: Data analysis details	87

Appendix K: Uncertainty analysis details	91

Appendix L: Metadata	92

# List of Tables

Table 1. List of Key Dimensions and Characteristics of the RTI F2 QD.	27

Table 2. Key Parameters at Full and 1/20th Scale	29

Table 3. Test Matrix	33

Table 4. Testing schedule	33

Table 5. QA Matrix	57

Table 6. Power and Data Channels	61

# List of Figures

Figure 1. General Schematic of bridge and MASK basin	14

Figure 2. General view of new segmented wavemaker in MASK Wavemaking Facility. Paddles are highlighted in red and the control cabinets are highlighted in bright blue	15

Figure 3. MASK carriage shown below the bridge at the center of the bridge	15

Figure 4. General wavemaker characteristics and design	16

Figure 5. MASK reference orientation, note that the orientation here is different than that in Figures 1 and 3	17

Figure 6. Self-orienting Elongated Wave Front Parallel Float RTI F2 QD 1/20th Test Configuration	18

Figure 7. Isometric view of 1/20th RTI F2 QD Test Configuration	19

Figure 8. Isometric RTI F2 QD Full Scale Model	19

Figure 9. RTI's Power Take-Off	21

Figure 10. Data Acquisition used during preliminary testing at UMO Alfond W2 Ocean Engineering Lab	22

Figure 11. Adjustable device flexibility to simulate full scale model	23

Figure 12. UMO Alfond W2 Ocean Engineering Lab	24

Figure 13. University of New Hampshire Chase Ocean Engineering Lab Floatation Testing	25

Figure 14. Key Characteristics of the RTI F2 QD design	26

Figure 15. RTI’s Motor Control Loop	32

Figure 16. RTI F2 QD anticipated mooring layout for testing at Carderock, MASK Basin (Dimensions in cm)	36

Figure 17. Side view of RTI's Mooring Configuration (Dimensions in cm)	37

Figure 18. Overhead view of RTI's Mooring Configuration - Watch circle and load cell locations (Dimensions in cm)	38

Figure 19. RTI E&C System Block Diagram – Processor Infrastructure	39

Figure 20. RTI E&C System Block Diagram - Swing-Arm Shaft Diagram	40

Figure 21. Ethernet Switch (Router) and DC Power Supply	43

Figure 22. Load Cell Connection to RJ50 Connector	44

Figure 23. Inverter, Resistor Brake and Brake Controller	44

Figure 24. CRio, Sensor Modules, AC Receptacle	45

Figure 25. Torque Control Inverter and the Piccolo TMS320F2806 Card	46

Figure 26. MATLAB GUI	47

Figure 27. Underwater view of RTI's Mooring attachment to the Heave/Surge Plate and Load Cell Location	48

Figure 28. Available Optical Tracking Locations	49

Figure 29. RTI F2 QD deployment and retrieval sub-assemblies	50

Figure 30. RTI F2 QD mooring deployment/retrieval system	51

Figure 31. Fully Assembled RTI F2QD with lifting cable at UMO Preliminary Testing	52

Figure 32.  RTI F2 QD mooring deployment/retrieval system	53

Figure 33. The RTI F2 QD is partially lifted from the bridge above until the entire PTO housing is above the SWL	54

Figure 34. Data flow and processing steps	56

Figure 35. RTI E&C System Block Diagram – Motor Shaft	61

Figure 36. RTI E&C System Block Diagram - Power Take-off	62

Figure 37. RTI E&C System Block Diagram - Miscellaneous Controls	62

Figure 38. RTI F2 QD 1/20th test model isometric	63

Figure 39. RTI F2 QD 1/20th scale detailed drawing – plan view	64

Figure 40. RTI F2 QD 1/20th scale detailed drawing – side view	65

This page intentionally left blank

# Introduction

The Department of Energy (DOE) launched the Wave Energy Prize (WEPrize) Competition as a mechanism to stimulate the development of new wave energy converter devices that have the prospect of becoming commercially competitive in the long run.  In the Final stage of the competition, nine teams will test their 1/20th scale devices at the US Naval Surface Warfare Center Carderock Division (NSWCCD) Maneuvering and Seakeeping Basin (MASK) in West Bethesda, MD.   Each contestant will prepare their device for one week and then test their device for one week at the MASK basin in Summer/Fall 2016.  This testing program will measure the performance of each device tested to determine the WEPrize winners.

The purposes of the Team Test Plan are to:

- Plan and document the 1/20th scale device testing at the Carderock MASK basin;

- Document the test article, setup and methodology, sensor and instrumentation, mooring, electronics, wiring, and data flow and quality assurance;

- Communicate the testing between the Finalist team, Carderock, Data Analyst (DA) and the Prize Administration Team (PAT);

- Facilitate reviews that will help to ensure all aspects (risk, safety, testing procedures, etc.) have been properly considered;

- Provide a systematic guide to setting up, executing and decommissioning the experiment.

The team test plan is a WEPrize required document and will be owned/managed by the Carderock Test Leads and DAs, and is intended to be a “living document” that will evolve continuously prior to the MASK basin testing.

# Test Objective

The top level objective of the 1/20th scale device testing is to obtain the necessary measurements required for determining Average Climate Capture Width per Characteristic Capital Expenditure (ACE) and the Hydrodynamic Performance Quality (HPQ), key metrics for determining the WEPrize winners [1].

# Test Facility

All testing will be conducted in the Maneuvering and Seakeeping basin (MASK) at Carderock Division, Naval Surface Warfare Center located in Bethesda, Maryland.  The MASK is an indoor basin having an overall length of 360 feet, a width of 240 feet and a depth of 20 feet except for a 35-foot deep trench that is 50 feet wide and parallel to the long side of the basin.  The basin is spanned by a 376-foot bridge supported on a rail system that permits the bridge to transverse to the center of the basin width as well as to rotate up to 45 degree from the centerline as seen in Figure 1.  Figure 1 does not include the physical update of this wavemaker system, but a drawing of the new paddle layout can be seen in Figure 2.  The MASK Carriage is suspended beneath the bridge and can travel along the rails by the rollers and drive system.  There is an arresting gear to prevent the carriage from hitting the end stops and this limits the travel along the bridge.  The carriage has 6’ x 10’ moon bay in the center which allows for models and instrumentation to be mounted.  A photo of the carriage is shown in Figure 3.  Along the two ends opposite of the wavemakers are beaches with a 12 degree slope. The beaches are constructed of 7 layers of concrete sections and are effective in mitigating the mass flux of water back into the tank during wave generation.  The hydrodynamic properties of the beaches can be found in [2].

Figure 1. General Schematic of bridge and MASK basin

Figure 2. General view of new segmented wavemaker in MASK Wavemaking Facility. Paddles are highlighted in red and the control cabinets are highlighted in bright blue

Figure 3. MASK carriage shown below the bridge at the center of the bridge

## Wave Maker

The new wavemaker is rendered with respect to its general install position in Figure 2.  The wavemaker system consists of 216 paddles.  There are 108 paddles along the North edge of the basin, 60 paddles in a ninety degree arc, and 48 paddles along the West edge of the basin.  The paddles are grouped in sets of eight paddles per control cabinet.  The 27 control cabinets are then joined via three marshaling cabinets, and ultimately the marshaling cabinets are connected to the main control station at the second floor of the MASK control room.  The cabinets and control room are generally illustrated in Figure 2.

A more detailed view of the wavemaker paddles is provided in Figure 4.  The paddles have a hinge depth of 2.5 m (8.2 ft.) and a pitch (centerline to centerline spacing) of 0.658 m (25.9 in.).  The wavemaker system is a dry back, force feedback system.  The paddles are moved using hydrostatic compensation with air tanks and bellows and with sectors attached to the wavemakers with an A-frame type structure.  The sector has a timing belt attached which runs on the topside of the sector.  The timing belt runs through a pulley box powered with an encoder controlled motor.  The motor is used to control the real-time quick motions of the paddle.  The force feedback of the paddle is provided via a force transducer mounted at the bellows and sector interface to the paddle.

The wavemaker is controlled via runtime software located on the main control computer using Edinburgh Designs Limited (EDL) software. The software allows entering specific regular wave conditions or it can be programmed to generate irregular seas via the input of “experiments files”.

Figure 4. General wavemaker characteristics and design

## MASK Orientation

With respect to the MASK basin, the reference frame is illustrated in Figure 5.  Its operational origin is located at the interior intersection of the northwest and northeast walls and vertically at the nominal 20 ft. water level.  The positive x-axis is aligned along the shorter northwest wall and the positive y-axis along the longer northeast wall.  Waves propagating parallel with the x-axis (toward the long beach) are defined as having a mean wave direction, β₀, of zero degrees and waves propagating parallel with the y-axis as 90 degrees.  This convention defines the wave direction as the direction the waves are traveling toward.

Figure 5. MASK reference orientation; note that the orientation here is different than that in Figures 1 and 3

# Scaled Model Description

## Device description

The RTI F2 QD consists of a self-orienting Elongated Wave Front Parallel (EWFP) Float.  The float is connected substantially below the Still Water Line (SWL) by two Drive or Swing Arms to a twin vertical spar semi-submerged buoyant frame as seen in Figure 6.   The rotatable arm to frame connection drives a rotary electric generator (single or dual) driven directly or through a gearbox at full scale in Figure 8.  A single rotary electric generator with gearbox is driven for the 1/20th scale model.  The EWFP Float is both lifted and driven rearward by the heave and surge forces, respectively, of oncoming waves, returning downward and forward, by gravity, during ensuing wave troughs. The Frame’s twin vertical spars are pivotably connected to a Submerged Mooring Buoy by two “lateral mooring lines” such that the F2 QD “weathervanes” (self-orients) about the Submerged Buoy maintaining the EWFP Float parallel to oncoming wave fronts.  The Submerged Mooring Buoy is affixed to mooring blocks on the tank bottom by three tensioned lines in Figure 7.  A detailed drawing of our 1/20th model is attached in Appendix A, Mechanical Section.

Figure 6. Self-orienting Elongated Wave Front Parallel Float RTI F2 QD 1/20th Test Configuration

Figure 7. Isometric view of 1/20th RTI F2 QD Test Configuration

Figure 8. Isometric RTI F2 QD Full Scale Model

## Power Take-Off description

The RTI F2 QD can utilize a single geared or direct drive multi-pole rotary electric generator Power Take-Off (PTO) driven by either one of the two Drive Arms producing power from wave forces which concurrently drive the Float both upward and rearward during each oncoming wave crest. Power is also produced on subsequent Float return strokes from gravity forces acting upon the Float as it moves downward and forward into subsequent wave troughs. Alternatively, because the RTI F2 QD uses a single Float (rather than fore and aft floats like the Columbia Stingray) two full or reduced sized geared or direct drive rotary generator PTOs can be utilized, one located in each of the two, port and starboard PTO housings providing redundancy in the event of a generator or gearbox failure.  Another alternative is to use one rotary generator PTO (again geared or ungeared) for power capture during wave force induced upstrokes and the other during subsequent gravity force induced down strokes by use of an over-riding or ratcheting clutch mechanism. This configuration allows each geared or direct drive rotary electric generator to always rotate in the same direction eliminating any gearbox or generator wear from reversing rotational direction during each wave crest and trough. This RTI F2 QD PTO configuration also allows short term (5-15 seconds) flywheel storage of captured wave energy smoothing generator electrical output.  In addition, it allows either PTO to provide full wave energy capture in the event of a generator or gearbox failure of the other PTO. While the RTI F2 QD can easily accommodate a large diameter multi-pole permanent magnet "pancake" type direct drive rotary electric generator, a planetary single stage geared generator reduces PTO CAPEX several fold. Direct drive 3-6 MW off-shore wind turbine generators cost $0.2-0.3 Million/MW and turn at 10-20 RPM. Direct drive Wave Energy Converters (WECs) with floats on swing or drive arms (like the RTI F2 QD, the Azura, and the Columbia StingRay) turn at 1-2 RPM and cannot afford $2-$3 Million/MW for their direct drive generators (unless the remaining WEC costs are free). Shortening swing or drive arms (relative to average wave height) does increase WEC RPM and PTO utilization somewhat (up to 2 fold maximum) but prior studies by U. of Edinburgh (Salter Duck and Sloped IPS Buoy) show that longer swing or drive arms result in higher wave energy capture efficiencies.

For the 1/20th Test Configuration, a multi-pole low RPM high torque permanent magnet AC Motor-Generator with an integral 8:1 gearbox is located in a water-tight housing within a submerged portion of the Starboard Frame Vertical Spar (Figure 9).  The drive axle is connected to the step-up gearbox of the motor-generator via a 70 tooth sprocket at the drive axle attached to a 15 tooth sprocket at the motor-generator by a #35 chain drive.  A resistive force (torque) is applied against oncoming waves by the EWFP Float as it is rotated by the Drive Arms and applies a lesser resistive force against the Float’s mass as gravity returns the Float (via reversing rotational direction of the Drive Arms) into the ensuing wave trough.

Figure 9. RTI's Power Take-Off

Two Motor-Generator torque control electronic systems (to increase low RPM Motor-Generator torque) are currently being utilized, including an AC voltage transformer and back driving the generator as a torque balancing motor.  In the event of electronics or software malfunction our backup torque control system uses our motor-generator as a 3-phase AC Generator connected to three manually adjustable load (0-100 Ohm) 100 watt resistors.

When the Float and its connected Drive Arm rotation reach 0 RPM (relative to the Frame) at each wave crest, the direction of rotation is reversed.   Gravity returns the float into the next ensuing wave trough while being resisted by a somewhat lower Motor-Generator programed and controlled torque. The Motor-Generator (through gearbox) Drive Arms are connected to a Common Drive Axle (to prevent accidental Arm twisting or Racking). Like the Columbia StingRay, the Full Scale RTI F2 QD will have sufficient Drive Arm and Float rigidity to avoid the need for a Common Drive Axle.

The Full Scale F2 QD Generator (with or without step-up gearbox) will be able to apply sufficient torque throughout each wave cycle to optimize energy capture (the Wave Energy Prize schedule did not allow time for the design, fabrication, and testing of a custom high torque generator-gearbox).

The F2 QD 1/20 model has two (2) RTI F2 QD tension and compression waterproof Load Cells located on each of the two (2) split Drive Arms in such manner that they will provide continuous real time Drive Arm Torque values for each Arm (which must be summed). The starboard side Cell will normally read higher than the port Cell because it is connected to the Motor-Generator while the port Drive Arm produces some torsional deflection of the long Drive Axel.  A high resolution magnetic encoder is located on the Motor-Generator (starboard) side of the Drive Axel. A second encoder on the port side will provide a redundant Real Time RPM value.  See Figure 10.

Figure 10. Data Acquisition used during preliminary testing at UMO Alfond W2 Ocean Engineering Lab

Torque and RPM real time output will be sufficient for determining instantaneous 1/20th scale F2 QD wave energy capture.

## Device properties

The model is being built light with the ability to increase Frame weight by over 50% and Float weight by over 100% by adding supplemental high density weights (2.5# and 5.0# steel  discs) to the weight studs on the Float fore Deck and on the Frame Drag Plate (either fore or aft of the Spar Leg center lines).  See Figure 11.

Figure 11. Adjustable device flexibility to simulate full scale model

The Center of Gravity (CG) of the Frame is located substantially below the Frame Center of Buoyancy (CB) and the Drive Axle to provide a sufficient restoring moment for Frame pitch recovery after the upper Frame (above CG) is pitched (rotated) rearward by the surge/lateral force of each oncoming wave. It is desirable, however, to have the Frame pitch recovery natural frequency substantially slower than the wave and Float (with Drive Arm) frequency such that the Frame is still pitching (rotating) forward (recovering from prior wave) while the Float is rotating (on its Drive Arms) rearward in response to the latest (current) wave. This lag between Frame pitch recovery and Float response to the current wave is evident in the 1/20th scale Test videos and in Figure 12 taken during WEP Preliminary Testing at University of Maine, Orono, Alfond W2 Ocean Engineering Lab and provides higher relative motion between Float and Frame and higher energy capture efficiencies.

Figure 12. UMO Alfond W2 Ocean Engineering Lab

The CG of the Float (including its rigidly attached Drive Arms) is located above and forward of the Float CB. This provides a slightly upward moment applied by the Float Arms to the Frame at the Axel location (raising frame slightly above the SWL without the Float attached) but more importantly provides a larger drive moment about the Drive Axel when the Float is returning via gravity into the next wave trough.

Adding or removing Float weight discs simulates the Full Scale F2 QD where the Float is partially flooded to provide a mass optimized capture efficiency for a given sea state (or deliberate de-tuning) or the Float is fully flooded to negative buoyancy allowing the Float to rotate on its elongated Swing/Drive Arms to a safe 6 o’clock position well below 15-20 meter wave troughs. The Float seawater ballast is partially or fully gravity drained simply using the PTO generator as a motor to lift the Float above the SWL while multiple Float Drain Valves are opened until the desired Float ballast tank water levels are obtained. During 1/20th scale testing under the “survival wave conditions” the Float interior volume will be substantially flooded to negative buoyancy with water through  the access port (w removable cover plate) on the Float Deck (using a section of tubing). The 1/20th Float is drained through the port by lifting the Float above the SWL with the Carderock lift.

The Drag Plate weight discs also simulate the Full Scale Frame which has compressed air cleared seawater ballast tanks within the Frame Floatation Chambers (above and below the PTO housings) on the Frame’s two Vertical Spars and an additional tank across the Horizontal Drag Plate (which also reinforces the Drag Plate).  Figure 13.

Figure 13. University of New Hampshire Chase Ocean Engineering Lab Floatation Testing

At full scale adjusting Frame seawater ballasts changes the Frame water line which raises or lowers the Frame Swing Arm Pivot Point (at PTO Drive Shaft) and Swing Arm neutral (still water) angle, CB, and CG. This either optimizes capture efficiency or provides de-tuning in heavy sea states.

Adding or removing Frame or Float mass at 1/20th scale using the weight discs also alters their center of buoyancy, center of gravity, and moment of inertia.  The actual values may be changed for each of the 10 Carderock Test Cases as Float weights and Frame weights are added or removed between tests.

Figure 14 depicts the Key Dimensions and Characteristics of the RTI F2 QD Design.  For an ‘As Built’ drawing of our 1/20th scale model, refer to Appendix A, Mechanical Section.

Figure 14. Key Characteristics of the RTI F2 QD design

Table 1. List of Key Dimensions and Characteristics of the RTI F2 QD.

|  | Measurement Reference Fig. 12 |  | Description | Comments | Verified at Carderock | Measured by Team |
| --- | --- | --- | --- | --- | --- | --- |
| Dimension (m) | (A) Overall Assembly Length (in direction of wave travel) fore to aft |  | 1.054 ± .0254 | Measured |  |  |
|  | (B) Width Overall (excluding PTO Housing Cap) |  | 1.829 ± .0254 | Measured |  |  |
|  | (C) Overall Assembly Height |  | 3.37 | Measured |  |  |
|  | (D) Float Assembly Overall Width |  | 1.400 ± .0254 | Measured |  |  |
|  | (E) Float Assembly Length (Fore to Aft) |  | .657 ± .0254 | Measured |  |  |
|  | (F) Float Assembly Height (excluding shoaling lip extension) |  | .235 ± .0254 | Measured |  |  |
|  | (G) Heave/Surge Plate Width |  | 1.834 ± .0254 | Measured |  |  |
|  | (H) Heave/Surge Plate Length (Fore to Aft) |  | .641 ± .0254 | Measured |  |  |
|  | (I) Length of Drive Arm |  | .800 ± .0254 | .445 m to edge of float - Measured |  |  |
|  | (J) Frame Twin Vertical Spar Centerline Width |  | 1.632 ± .0254 | Measured |  |  |
|  | (K) Drive Axle to Drag Plate Depth |  | 2.17 ± .5 | Measured |  |  |
|  | (L) Drive axle to SWL |  | .343 ± .1 | Measured |  |  |
| Please provide the following measurements for each body of the WEC |  |  |  |  |  |  |
| Overall Mass (kg) |  |  | 105 ± 2.27 | Without added mass for ballast |  |  |
| Overall Buoyancy (kg) |  |  |  | Variable by design |  |  |
| Overall Center of gravity (m) |  |  | .105 | -.038 | -.4343 |  |
| Overall Center of buoyancy (m) |  |  |  | Variable by design |  |  |
| Moment of inertia system |  | Pitch (kgm2) | 1.605E+6 | Calculated |  |  |
|  |  | roll (kgm2) | 1.176E+6 | Calculated |  |  |
|  |  | yaw (kgm2) | 5.447E+5 | Calculated |  |  |
| Mass of float (kg) with drive arms |  |  | 26.04 ± 2.27 | Measured |  |  |
| Buoyancy of float (kg) |  |  |  | Variable by design |  |  |
| Center of gravity of float (m) |  |  | .3948 | -.006 | -.0723 |  |
| Center of buoyancy of float (m) |  |  |  | Variable by design |  |  |
| Moment of inertia float |  | Pitch (kgm2) | 9.62E+00 | Calculated |  |  |
|  |  | roll (kgm2) | 7.58E+00 | Calculated |  |  |
|  |  | yaw (kgm2) | 1.21E+01 | Calculated |  |  |
| Mass of frame (kg) |  |  | 54.41 ± 4.5 | Measured |  |  |
| Buoyancy of frame (kg) |  |  |  | Variable by design |  |  |
| Center of gravity of frame – rel. to SWL (m) |  |  | .55 Z direction | Calculated |  |  |
| Center of buoyancy of frame (m) |  |  |  | Variable by design |  |  |
| Moment of inertia frame |  | Pitch (kgm2) | 6.39E+01 | Calculated |  |  |
|  |  | roll (kgm2) | 2.84E+01 | Calculated |  |  |
|  |  | yaw (kgm2) | 3.58E+01 | Calculated |  |  |
| Mass of heave plate (kg) |  |  | 18 ± 2.27 | Measured |  |  |
| Buoyancy of heave plate (kg) |  |  |  | Variable by design |  |  |
| Center of gravity of heave plate (m) |  |  | -2.43 | Measured |  |  |
| Center of buoyancy of heave plate (m) |  |  |  | Variable by design |  |  |
| Moment of inertia heave plate |  | Pitch (kgm2) | 6.22E+00 | Calculated |  |  |
|  |  | roll (kgm2) | 9.43E-01 | Calculated |  |  |
|  |  | yaw (kgm2) | 6.31E+00 | Calculated |  |  |
| Operating Draft (based on SWL to lowest point of device) (m) |  |  | 2.59 ± .5 | Measured |  |  |
| Wetted surface area (m2) |  |  | 12.88 | Calculated |  |  |

## Froude scaling

The geometry of the device was scaled linearly with the scale factor, s=20. Mass values were scaled with s3, and mass moments of inertia with s5. Since the power take-off is rotational, the kinematic component (the relative angular velocity between the float and the frame) scales with s-0.5 and the dynamic component (the torque on the drive shaft) scales with s4.

For certain parameters (e.g. the thickness of the steel heave-surge Drag Plate and the aluminum Float’s skin), Froude scaling from Full Scale to our 1/20 model would result in dimensions somewhat thinner than those practical at model scale. In these cases, the mass and location of high-density ballast will be adjusted to account for the discrepancy in mass values.

Table 2. Key Parameters at Full and 1/20th Scale

## Other scaling or measures

The submerged heave and surge plates will be subject to Froude number, Keulegan–Carpenter number, and Reynolds number dependent flow dynamics. Keulegan–Carpenter number scales directly with Froude number, so the ratio of form drag to inertial forces is conserved.

Unlike Keulegan–Carpenter dependent effects, Reynolds number effects will not scale properly on a Froude scaled model. However, the error associated with this discrepancy will be mitigated by the radiused edges of the heave and surge plates. This rounding will reduce eddy losses that contribute to Reynolds number sensitivity of the dynamics. Furthermore, the exaggerated effect of viscosity at model scale will have the effect of reducing the power absorbed by the device.

Numerical modeling—validated by test data gathered at the Small Scale Testing Facility during Technology Gate 2—shows that the float displacement amplitudes are generally larger than the corresponding wave-induced fluid motions. Thus, viscosity has a net retarding effect on the float motion. So the artificially low Reynolds number at model scale will result in unrealistically high amounts of energy being dissipated from the float by viscous effects.

The frame is designed to lag the float motion by 90 o–180o. So the reaction force that it provides to the float and PTO is proportional to its inertia. Excessive viscous effects will dissipate energy from the frame motion, decreasing its inertia. This will reduce the dynamic component of the PTO, reducing absorbed power.

As stated in Section 4.3 (Device Properties) above, the RTI F2 QD 1/20th scale model Float and Frame are deliberately being built light with the ability to increase Float mass by up to 100% and Frame mass by up to 50% to conform 1/20th  scale and Full Scale Froude scaling of Mass, CG, CB, and Moment of Inertia. Because the F2 QD Full scale model also has the on-board adaptive control ability to make major changes in Float Mass (up to 400%) and Frame Mass (25%-50%) with its seawater ballast tanks, the 1/20th  Model and Full Scale design mass can readily be brought into Froude conformity. Float and Frame dimensional changes made in the 1/20th scale model will result in comparable Froude scaled changes in the Full Scale design.

## Control strategy

RTI is utilizing vector control, also called field-oriented control (FOC) during the 1/20th scale testing.  It is a variable-frequency drive (VFD) control method in which the stator currents of a three-phase AC electric motor (a permanent magnet brushless motor in our case) are identified as two orthogonal components that can be visualized with a vector. One component defines the magnetic flux of the motor, the other defines the torque. The control system of the drive calculates the corresponding current component references from the flux and torque references given by the drive's control. Typically proportional-integral (PI) controllers are used to keep the measured current components at their reference values. The pulse-width modulation of the variable-frequency drive defines the power transistor switching according to the stator voltage references that are the output of the PI current controllers.

The control loop shown in Figure 15 is implemented in software to read the necessary data from the current and voltage sensors on the drive board and estimate flux, motor angle, motor speed and torque. The estimated data is compared to the reference values of these variables in the control loop and the necessary correction in the PWM switching pattern is executed in terms of microseconds.

The speed of the system is multiplied by a control gain and the result becomes the torque reference for the FOC torque control loop.  See Figure 15.

Figure 15. RTI’s Motor Control Loop

Our primary motor-generator torque control system which electronically back drives our motor-generator as a motor using control software can apply higher torques when the float is rising on wave crests (due to wave forces) and a different (generally lower) torque when the float is returning (via) gravity into ensuing wave troughs.  Again, in the event of electronics or software malfunction our backup torque control system is to use our motor-generator as a 3-phase AC Generator connected to three manually adjustable load (0-100 Ohm) 100 watt resistors.

The RTI F2 QD 1/20th scale Control Strategy is Summarized in Section 4.2 (Power Take-Off Description).  Section 4.3 Device Properties,  describes how Adaptive Controls (changing Float mass and Frame Mass) will be implemented during 1/20th scale RTI F2 QD testing (by changing Float and/or Frame weight discs between tests)  and at Full Scale (adding or removing seawater ballast from the Float and/or Frame ballast tanks).

The energy efforts utilized for adaptive control during Carderock testing are different from the adaptive control energy efforts for the full scale F2 QD.  During 1/20th testing, float and/or Frame weights will be changed and mid and lower vertical spars may be manually drained.  At full scale, the Frame uses tank stored compressed air (produced with on-board compressors) to displace water out of the ballast tanks and the Float is drained via lifting above the SWL.  Even if the Full Scale adaptive controls (Lifting and partially draining Float or displacing a small amount of Frame seawater ballast with compressed air) are utilized 2-3 times/week, the energy consumption of these controls are insignificant relative to energy production (but calculable). Waves lift the Float above the SWL (where it can drain some ballast water) every 6-16 seconds. Float lifting energy can be eliminated or minimized by simply locking the Float when in a position above the SWL (via using either resisting generator torque or a mechanical ratcheting or electromechanical brake/clutch means) and draining Float water ballast (out of valve controlled drains) until the desired new Float water ballast level is obtained and energy production is resumed. Frame ballast increases (to lower the Frame and Drive Arm pivot point) require no energy (except opening an air exhaust valve in the top of a ballast tank which allows water to enter through an always open bottom aperture). Re-elevating the Frame (to elevate the Drive arm pivot point) is accomplished by discharging a sufficient volume of compressed air (from the onboard compressed air tanks or compressors) through a control valve into the top of the ballast tanks, until the desired new Frame SWL has been reached. The energy consumed is the air compression work utilized to replace the quantity of compressed air utilized.

# Test Matrix and Schedule

## Test matrix

The incident wave conditions for the 1/20th scale experiments at NSWC Carderock’s MASK are shown in Table 3. .  Carderock will perform wave environment calibration in summer 2016. The result of this calibration is shown in Appendix E.

Table 3. Test Matrix

| Type | Number | TP [s] | HS [m] | γ (gamma) | Direction | Spreading |
| --- | --- | --- | --- | --- | --- | --- |
| IWS (JONSWAP) | 1 | 1.63 | 0.117 | 1.0 | 10.0 | ∞ |
|  | 2 | 2.20 | 0.132 | 1.0 | 0.0 | ∞ |
|  | 3 | 2.58 | 0.268 | 1.0 | -70.0 | ∞ |
|  | 4 | 2.84 | 0.103 | 1.0 | -10.0 | ∞ |
|  | 5 | 3.41 | 0.292 | 1.0 | 0.0 | ∞ |
|  | 6 | 3.69 | 0.163 | 1.0 | 0.0 | ∞ |
| LIWS (JONSWAP) | 7 | 3.11 | 0.395 | 3.3 | -30.0 | 3.0 |
|  | 8 | 2.50 | 0.460 | 3.3 | -70.0 | 7.0 |
| RWS (4-parameter JONSWAP) | 9 | 3.22 | 0.076 | 2.0 | -70.0 | 7.0 |
|  |  | 1.61 | 0.108 | 2.0 | 0 | 10 |
|  | 10 | 3.32 | 0.079 | 2.0 | -70.0 | 7.0 |
|  |  | 1.93 | 0.065 | 2.0 | -10 | 10 |

## Test schedule

Table 4. Testing schedule

| Date/Time | Event |  |
| --- | --- | --- |
| Monday | MASK installation and work-in |  |
| 7:00 | Morning Huddle |  |
| 7:10 | Contestants will be moving their device from the assembly area to the installation area, installation and verifying operation |  |
| Tuesday | Continued installation and work-in |  |
| 7:00 | Morning Huddle |  |
| 7:10 | Contestants continue moving their device from the assembly area to the installation area, installation and verifying operation |  |
| 2:00 | Readiness verification |  |
| 3:00 | Baseline 1 run | IWS Wave 2 |
| 4:00 | Baseline 2 run | IWS Wave 5 |
| 4:20 | Contestants pack up for evening |  |
| Wednesday | Full Test Day |  |
| 7:00 | Morning Huddle |  |
| 7:15 | Contestants set up for testing and perform pre-test checks |  |
| 8:00 | Run 1 (Baseline 1) | IWS Wave 2 |
| 9:00 | Run 2 (Baseline 2) | IWS Wave 5 |
| 10:00 | Run 3 | IWS Wave 1 |
| 11:00 | Run 4 | IWS Wave 3 |
| 12:00 | Lunch |  |
| 1:00 | Check Run 1 (Baseline 1) | IWS Wave 2 |
| 2:00 | Run 5 | IWS Wave 4 |
| 3:00 | Run 6 | IWS Wave 6 |
| 4:00 | Check Run 2 (Baseline 2) | IWS Wave 5 |
| 5:00 | Contestants pack up for evening (also a 30 minute buffer) |  |
| Thursday |  |  |
| 7:00 | Morning Huddle |  |
| 7:15 | Contestants set up for testing and perform pre-test checks |  |
| 8:00 | Check Run 3 (Baseline 1) | IWS Wave 2 |
| 9:00 | Run 7 | RWS Wave 1 |
| 10:00 | Run 8 | RWS Wave 2 |
| 11:00 | Run 9 | LIWS Wave 1 |
| 12:00 | Lunch |  |
| 1:00 | Run 10 | LIWS Wave 1 |
| 2:00 | Check Run 4 (Baseline 2) | IWS Wave 5 |
| 3:00 | Backup Run 1/ Contestant Testing | TBD |
| 4:00 | Backup Run 2/ Contestant Testing | TBD |
| 5:00 | Perform final daily data QA checks and test reporting (may start earlier if testing permits) |  |
| Friday |  |  |
| 7:00 | Morning Huddle |  |
| 7:15 | Contestants set up for testing and perform pre-test checks |  |
| 8:00 | Backup Run 3/ Contestant Testing | TBD |
| 9:00 | Backup Run 4/ Contestant Testing | TBD |
| 10:00 | Backup Run 5/ Contestant Testing | TBD |
| 11:00 | Backup Run 6/ Contestant Testing | TBD |
| 12:00 | Lunch |  |
| 1:00 | Contestants pack up for shipping |  |

Post Test Notes for Section 5.2

The test schedule was modified and all test waves were run. Please see the test results spreadsheet for a list of waves that were run and the order for which they were run.

# Experimental Set Up and Methods

## Mooring

Figure 16. RTI F2 QD anticipated mooring layout for testing at Carderock, MASK Basin (Dimensions in cm)

Figure 17. Side view of RTI's Mooring Configuration (Dimensions in cm)

Figure 18. Overhead view of RTI's Mooring Configuration - Watch circle and load cell locations (Dimensions in cm)

## Instrumentation

Figure 19 and Figure 20 depict the primary Processor Infrastructure and the Swing (Drive) Arm Shaft Control Strategy.   The remainder of the Electronics and Controls (E&C) Block Diagrams can be reviewed in Appendix A, Electrical Section.

Figure 19. RTI E&C System Block Diagram – Processor Infrastructure

Figure 20. RTI E&C System Block Diagram - Swing-Arm Shaft Diagram

RTI F2 QD Electrical Test Setup Procedure

Figures:

Figure 21. Ethernet Switch (Router) and DC Power Supply

Figure 22. Load Cell Connection to RJ50 Connector

Figure 23. Inverter, Resistor Brake and Brake Controller

Figure 24. CRio, Sensor Modules, AC Receptacle

Control and Sensing Connections:

- Laptop, Ethernet Switch, DC Power Supply

  - Laptop

    - Ethernet connection to Ethernet switch (either port)

    - USB to MicroUSB for programming break-out boards on FPGAs

  - Ethernet switch Ethernet connection to third from the left CRio slot (IP Address: 10.0.0.3)

  - DC Power Supply require power from AC

    - Set Voltage to 38VDC

    - Connect green lead from load end of cable to positive input on Invertor

    - Connect white lead from load end of cable to gnd input on Invertor

- CRio and Sensor Modules Rack

  - CRio in Slot 1 of Rack

    - Connect Power from AC receptacle and Ethernet cable from Ethernet Switch

  - Load Cell Module in Slot 6 of Rack

    - Port sensor in slot 2 (RJ50 Cables*). Starboard sensor in slot 3.

    - Power from AC receptacle.

*RJ50 Cables required by NI for connection to sensor modules.

  - Encoder Module in Slot 8 of Rack

    - Soldered connections (left on WEC unit).

    - AC receptacle power also soldered.

- Inverter, Break, and Break Controller

  - Inverter

    - Green and White leads connected from DC power.

    - Green, black, and white leads 3-phase connections from Generator output

    - MicroUSB from Laptop remains connected for the duration of data gathering after programming break control.

  - Brake Controller

    - DC Link (Red and White leads) from Positive and GND connection of FPGA board to Positive and GND connection of FPGA invertor board.

      - DC Link provides power to the FPGA boards.

  - Resistor “Brake”

    - Connected to phase A and GND on FPGA board.

Software Setup:

- CCS – Compiler

  - Program Break

    - MicroUSB connected to Break Controller.

    - Compile by pressing the Debug button.

    - Press the Run button on upper left. Check for flashing red LED indication on break-outboard. Remove MicroUSB cable and switch MicroUSB cable to invertor controller.

    - Stop program

  - Program Inverter

    - Double click Project Lab 04A.

    - Debug, Run. Set on continuous refresh by press the gold button on the upper left.

- MATLAB

  - Run to Open GUI

    - GUI

      - Starts / Stops Inverter (Need to “Start” to turn on Inverter)

      - Set Gain to -25 (negative indicates generating, positive indicates motoring). Press button to set the gain after inputting numerical value.

- LabView

  - Run RTI DAQ File.

  - Connect to CRio by right clicking on CRio object line on left hand column.

  - Open file called State Machine.VI

  - Set parameters as shown in solid blocks.

  - To save data collected instantaneously, enter file name ending prior to start of data collection.

  - Click the Run continuous button to run the VI prior to clicking Run (to collect data on the user interface).

  - Check to make sure data is outputting graphically.

- Retrieve LabView Files

  - Click “ThisPC” in file explorer.

  - Click “Map Network Drive” in menu bar.

  - Within Folder field, type http://10.0.0.3/files

  - Check “connect using different credentials”. Press finish.

  - Enter the following credentials Username:Admin and Password: LabView

  - Root folder. Root/C/Test_Results

Figure 21. Ethernet Switch (Router) and DC Power Supply

Figure 22. Load Cell Connection to RJ50 Connector

Figure 23. Inverter, Resistor Brake and Brake Controller

Figure 24. CRio, Sensor Modules, AC Receptacle

Software Description

The controls platform is comprised of three separate pieces of software.  The three separate pieces of software are uploaded into the DOE WEC prize folder. The file names of the software are as follows: WaveGUI.zip, Torque_Controller_software.zip, and active_brake_software.zip.

Active Brake Software (active_brake_software.zip)

The active brake software runs on a TI microcontroller embedded in an inverter board.  The sole purpose of this board is to maintain energy balance on the DC bus.  In short, this software commands the inverter to consume the power produced by the wave energy converter.  The energy is converted to heat by varying the voltage applied to a resistor.  This software has two adjustable set points.  The first set point is the voltage of the DC bus where the brake engages.  The second set point is the DC bus voltage where the brake is fully engaged.  There is a linear progression between the set points where the brake engages from 0% to 100%.  That is to say, beyond the second set point the brake is 100% engaged, below the first set point the brake is 0% engaged, and between the two set points the brake is engaged linearly from 0-100% depending on where the DC bus is between the two set points.

The two set points in the active brake software that we will adjust are _IQ(0.041) and _IQ(0.042).  These are by default set to 41 Volts and 42 Volts in IQ24 kilo-Volt format.

Torque Controller Software (Torque_Controller_software.zip)

The torque controller software runs on another TI microcontroller embedded in an additional inverter board.  The purpose of this software is to produce the amount of torque requested by the control loop.  The control loop is measuring the speed of the motor/generator.  The speed is multiplied by a gain and the result of this multiplication is the requested reference torque.  There are two different gains that can be used to multiply the speed by and produce the torque reference.  The two variables are ControllerGain_pos and ControllerGain_neg.  The gain that is used depends on the sign of the speed. If the speed is positive, then it is multiplied by ControllerGain_pos resulting in the torque reference.  If the speed is negative, then it is multiplied by ControllerGain_neg resulting in the torque reference.  Both variables ControllGain_pos and ControllerGain_neg are adjustable through serial link communications.

Wave GUI (WaveGUI.zip)

Wave GUI is used as a means to modify the necessary variables on the TI microcontroller which is controlling the motor/generator inverter via a serial link interface. Code composer studio is used to program and view the Piccolo TMS320F2806 on the inverter for the brake and the inverter for the torque controller.

Figure 25. Torque Control Inverter and the Piccolo TMS320F2806 Card

Figure 25 is depicts the WaveGUI interface.  The explanation of buttons and inputs is below.

• Start and stop buttons are for turning on and off the Torque controller Inverter.

• Two separate gains are used to control the torque command of the inverter in both directions independently.  The Set button is used to apply the changes in gain section.  These modify the variables ControllGain_pos and ControllerGain_neg.

• Check Communication button is for testing the link between MATLAB Wave GUI and the TI microcontroller.

• COM Port defines the port that the GUI uses to send the serial commands to the Piccolo TMS320F2806 card.

• Delete COM Objects button is used to clear serial objects in MATLAB

Figure 26. MATLAB GUI

Carderock load cell Tecsis XLU68F attachment locations are depicted in Figure 27 below.

Figure 27. Underwater view of RTI's Mooring attachment to the Heave/Surge Plate and Load Cell Location

Post Test Notes for Section 6.2

During the spot checks, the port load cell failed the check while the starboard load cell, and both port and starboard encoders passed the checks. Thus, the port load cell was removed and only the starboard arm transmitted load. Thus, only the toque measurement from the starboard load cell was used to calculate the dynamic side of power.

## Motion tracking

A natural point tracking system will be used to track the motion of the platform. The optical tracking locations are shown in figure 28.

Figure 28. Available Optical Tracking Locations

## RTI F2 QD Deployment and Retrieval Summary

The RTI F2 QD will be shipped, disassembled and assembled from the following major sub-assemblies (Figure 29);

Vertical Frames (two) with PTO Housings (Fiberglass/Foam floatation attached to housing) and attached Mid Frame section. The Motor-generator, chain drive, encoder, 1.0” Drive Axel stub-sections, and mini-bilge pump are located within the PTO Housing. The Electro Magnetic Clutch (EMC), Drive Axel Stub Shaft, and 2nd encoder are located in the EM Housing. The Lower Frame legs may also be removed during transit (4 bolt non-water tight flanged connections).

Aluminum sheet metal Float assembly (sealed and water-tight)

Drag Plate assembly including Horizontal and Vertical Drag Plates, 4 HD Ballast Studs/pins, and telescoping mount to lower vertical Frame legs (allowing quick out-of-water-change in Drive Axel to Drag Plate dimension). Will not be changed during testing.

Drive Axel with port and starboard (2) Drive Arms attached (with 1” bore clamp coupling).

Upper Lateral (horizontal) Frame Cross-Truss (4 bolt Flanged connections to Upper vertical Frame sections). All control wires go through Truss to mid-Truss take-off point. A plastic coated steel lifting cable (connected to top of Vertical Frame sections, with centered lifting loop, provides a single lift point.

Figure 29. RTI F2 QD deployment and retrieval sub-assemblies

- The mooring system in Figure 30 consists of one submerged (9” X 14”) mooring buoy, 3 connected diverging oblique (450 to 600) mooring lines (connected to Carderock supplied 140 lb. mooring blocks), and two (2) Ball to F2 QD Lateral Mooring lines. The mooring ball also has a “swivel lateral plate” with a vertical post or hook for attaching the two (2) converging F2 QD frame attached “Lateral” mooring lines to the plate and mooring ball.

Figure 30. RTI F2 QD mooring deployment/retrieval system

- Deployment

The Mooring system is assembled and deployed first (see Figure 30). It is suggested that the 3 Carderock provided mooring blocks first be pre-deployed with a small surface buoy marking the location of each block. A small Toggle float is provided to locate and set the Submerged Mooring Buoy at its proper depth (1.1 meters to Swivel Plate). The Carderock mooring blocks/anchors can be moved slightly inward or outward (from target 450 mooring line angle) to set the RTI Submerged Mooring Buoy near 1.1 meter.

The fully assembled F2 QD is then lifted (using its steel cable with single lifting loop) into the wave tank (approximate weight is 110 kg). See Figure 31.  The two Lower Frame sections (below the PTO Housing) have drain plugs and are pre-filled with correct water ballast and before deployment (Mid-Frame filled with air and Lower Frame with water).

Figure 31. Fully Assembled RTI F2QD with lifting cable at UMO Preliminary Testing

The assembled F2 QD is floated into position and its two (2) “Lateral Mooring Lines” (each approx. 2.5 meters long) converging at a Lateral Swivel Plate with mounting hole is dropped over a single vertical curved Catch-Release Pin on a lateral bar (also swiveling) attached to the Buoy top using a minimum 8 ft. long boat hook. A small monofilament loop on the top of the Swivel Plate allows future detachment (with a boat hook) of the Lateral Lines and Swivel Plate from the Pin.  See Figure 32.

Figure 32.  RTI F2 QD mooring deployment/retrieval system

Float and Frame (Drag Plate) high density Weights are added as described in Section C below.

Finally the Control Lines coming from the middle of the upper Lateral Cross-Truss are attached to the RTI control system and WEP Data Acquisition systems on the overhead Carderock Bridge.

B.  Retrieval steps use the Deployment steps in reverse order.

Detachment and Partial Lift of RTI F2 QD Nightly between Test Days (to prevent damaging overnight water intrusion into the PTO Housings when the unit and its mini bilge pumps are unpowered).

The two (2) Lateral Mooring Lines from the F2 QD port and starboard vertical Frame members (or ends of Drag Plate) converging at the Mooring Ball Swivel Plate is lifted off  the post on the top of the Submerged Mooring Buoy using a boat hook.

The F2 QD is partially lifted from the bridge above until the entire PTO housing and the Float is above the water line (about 0.6 meter lift) and secured with 1/4” or 5/16” line to the bridge as in Figure 33 (or suspended from an overhead crane hook). The electrical umbilical need not be detached.

Figure 33. The RTI F2 QD is partially lifted from the bridge above until the entire PTO housing is above the SWL

- Protocols for Adding or Subtracting High Density Metal Ballast Weights from RTI F2 QD Float and/or Frame between MASK Tests (to simulate the patented Float and Frame seawater ballasting of the Full Scale RTI F2 QD)

Painted steel 2.5# and 5.0# discs with a center hole and monofilament lifting loop (up to 18 cm diameter and up to 2.5 kg each) will be used interchangeably as either Float Weights or Frame Weights (placed on the vertical Drag Plate or Float Weight Posts).

Adding or subtracting Float Weights can be done by hand from the MASK punt (or using an 8-12 ft. boat hook from the Bridge). Float Weights are placed or removed from the 2 Weight Studs-Posts on the Front port and starboard sides of the Float Deck. Retaining 3/8” wing nuts are available but only utilized during the more severe wave test conditions.

There are 4 Weight Studs/Posts located on the 4 corners (port and starboard, fore and aft) of the Horizontal Drag Plate. Each weight disc has a monofilament lifting loop which can be captured via a boat hook and lifted off or placed on its Weight Stud-Post. There is no need to use wing nuts on these weights as Drag Plate motion, even with severe waves, is quite docile. Because the Horizontal Drag Plate is 2.0 meters below the Still Water Line a telescoping (to 10 ft. length) Boat Hook is used. A sight glass is also provided by RTI to eliminate sight refraction and get a good view of the Weight lifting loops when using the boat hook (especially when water surface ripple and/or glare is present).

# Data Processing and Analysis

## Data quality assurance and on-site processing

Data collection will start 2 minutes before waves are started and continue for at least 2 minutes once wave generation stops. This will ensure that the data captures the initial conditions and ramp-up/down.

“Raw” data from the Natural Point motion tracking and from the National Instruments (NI) measured power/loads/other are collected on two different systems and stored in separate text files. The motion tracking data are stored in a CSV file while the data from NI DAS are stored in a tab delimited text file.

## Data analysis

The data processing and analysis is divided into two parts: 1) data quality assurance (QA) that will ensure that quality, consistent and error free data are used in data analyses and 2) data analysis to calculate the performance metrics used in judging.  The data flow and processing steps are shown in 17 and the calculated values for the data analysis are provided in Appendix J.

Post Test Analysis

Wave Sensors

Motion Tracking System

Mooring Loads Sensors

WEC PTO Sensors

Carderock DAS

Write to optical Disc

Signal Conditioning

Unit Conversion

Data Formatting

DAS Real Time Data Display

Test-by-Test Data Analysis

Signal Conditioning and Unit Conversion

Initial QA checks

Initial Processing

Secondary QA checks

Test Report

Contestant Controller

Processing and analysis

DA Test Data Display

Figure 34. Data flow and processing steps

The objective of the data quality check is to detect and eliminate as many significant errors from the data as soon as possible, and to come to an overall assessment of the data quality.  The data QA shall be performed at three points during testing: 1) visually in “real time” during each test while data are collected, 2) during the interval after testing when the wave basin in settling and 3) when data are analyzed. It is critical to identify any data issues as soon as possible so corrective action can be taken and a test rerun if necessary.

The DAs and Carderock will decide if a test needs to be rerun if they have determined the data is of sufficiently poor quality in terms of:

- The wave field did not sufficiently match the specified spectrum

- There were errors in the measurements due to such issues as sensor failure, connector failure, too high noise, etc.

- Failure or issues with the WEC

- Fault with DAS

### “Real Time” Data QA

To ensure data quality, to prevent re-running multiple tests, and to halt tests early, all channels shall be visually monitored during testing to provide a basic level of data quality assurance and to verify that all instruments and the data acquisition system are functioning properly. If bad data are detected, the test lead should be immediately notified, who will then decide what action needs to be taken. During each test the following QA should be performed:

- Operation and performance of the DAS should be monitored to verify that it has not locked up or faulted – make sure the DAS runs throughout the test by monitoring CPU load and data updates

- Visual inspection of the data being displayed by the DAS, as they are gathered – the Carderock DAS will plot specific incoming data channels as they are acquired.

### Settling Interval and Time between Test Data QA

After each test, while the basin settles and while the next test is set up (~20 min total), a more detailed data QA shall be performed to identify any issues before the next test starts. The DA will do their best to perform this task between runs, but if this is not possible, the QA will be completed during the subsequent run. If issues are detected with the data, these will be brought up to the test lead. The following tasks should be performed:

- Operation and performance of the DAS should be monitored to verify that it has not locked up or faulted

- Time series for each data channels should be plotted and inspected

- Data shall be processed to perform higher-level data QA

- Spectra should be calculated for waves, power and loads and plotted and inspected

- Wave spectra should be compared with baseline wave spectra

- Periodic comparisons with baseline runs (as possible)

- Visual inspection of all wires, connectors and sensors should be performed (as possible)

- Visual inspection of device should be performed (as possible)

The first six bullets will be performed using pre-written scripts that interface with the Carderock DAS storage. These scripts will load the data, perform some processing, create figures for review, and identify any data of concern.

Table 5. QA Matrix

|  |  | Real Time via observation | Settling Interval | Post Test |
| --- | --- | --- | --- | --- |
| DAS malfunction | Check for data acquisition failure or malfunctions | X | X |  |
| Sensor malfunction | Check for sensor failure or malfunctions | X | X |  |
| Time difference | Check the time difference between each measurement for consistency and against specifications and check for strange variations in time |  | X | X |
| Error values/substitutes | Identify error values/substitutes (i.e. “999” or “NaN”) | X | X | X |
| Constant value | Repetitions of consecutive data with the same value (repeating standard deviations or offset). |  | X | X |
| Completeness | Check whether the number of records and their sequence is correction (identification of gaps, check for repetition) |  |  | X |
| Range /threshold | Check whether the data of each sensor lie within the measurement range of that sensor. |  | X | X |
| Measurement continuity | Compare the rate of change of a signal to expected/seasonally accepted values and between similar measurements that are collocated in close proximity |  | X | X |
| Measurement Consistency | comparison between statistics, such as the ratio of wave height to power |  |  | X |
| Near-by Comparison | Comparison with similar/duplicate measurements that are collocated in close proximity |  |  | X |
| Spectral spikes | Spikes in the spectral data |  | X | X |
| Trends and inconsistencies | Identify trends in data such as large drift in sensor output or inconstancies in sensor output for similar input |  |  | X |

# Data Management

Data are transferred from the Caderock systems (Natural Point and the NI DAS) to the DA computers via an optical media, likely a re-writable DVD or Blu-ray. Each disc will be labeled with the data, the team name and the included runs. Separate discs will be used for each team.

Data will be transferred to the DA computer and stored in separate directories for each team. As data are processed, the processed data, along with the processing algorithms will be stored on the DA computers. The “raw” data files SHALL not be altered by the DAs – if modifications are needed, a new file shall be created to do this, thus, preserving the original “raw” data file.

At lunch and at the end of each day, all data will be backed up to two separate hard drives and to the spare DA computer. One drive will remain at Carderock and the other will be stored at a separate location during evenings and weekends.

When DAs are using computers other than the DA computers, all analysis and algorithms will be backed up to two different jump drives at least once a day. Once analysis is complete, the DA will send one drive to the lead DA who will archive the raw and processed data on NREL’s secure data server.

The teams will not be provided with any data from Carderock or the DAs – the PAT will facilitate data transfer to the teams.

The discs, DA computer, the redundant back-up drives, and storage on NREL’s secure server provide a high level of storage redundancy.

# Appendix A: Device electrical and mechanical drawings

Electrical Section

Table 6. Power and Data Channels

Figure 35. RTI E&C System Block Diagram – Motor Shaft

Figure 36. RTI E&C System Block Diagram - Power Take-off

Figure 37. RTI E&C System Block Diagram - Miscellaneous Controls

Mechanical Section

Figure 38. RTI F2 QD 1/20th test model isometric

Figure 39. RTI F2 QD 1/20th scale detailed drawing – plan view

Figure 40. RTI F2 QD 1/20th scale detailed drawing – side view

# Appendix B: PTO calibration results

# Appendix C: Device Froude scaling

| Quantity | Froude Scaling | Reynolds Scaling |
| --- | --- | --- |
| wave height and length wave period and time wave frequency power density | s s0.5 s-0.5 s2.5 | s s2 s-2 s-2 |
| linear displacement angular displacement | s 1 | s 1 |
| linear velocity angular velocity | s0.5 s-0.5 | s-1 s-2 |
| linear acceleration angular acceleration | 1 s-1 | s-3 s-4 |
| mass force torque pressure power | s3 s3 s4 s s3.5 | s3 1 s s-2 s-1 |
| linear stiffness angular stiffness | s2 s4 |  |
| linear damping angular damping | s2.5 s4.5 |  |

Froude scaling R (Load), where lower case r is model scale and capital R is full scale:

Dynamic (force) to kinematic (velocity):

rR=fFVv=s3∙s-0.5=s2.5

Dynamic (torque) to kinematic (angular velocity):

rR=τΤΩω=s4∙s0.5=s4.5

Dynamic (pressure) to kinematic (volumetric flow):

rR=pPQq=pPVvL2l2=s∙s-0.5∙s-2=s-1.5

.

# Appendix D: Detailed description of control strategy

For the Detailed Description of the RTI F2 QD Control Strategy please first refer to Sections 4.2 PTO Description, 4.3 Device Properties, and 4.6 Control Strategy. Please also refer to the E&C (Electronics & Controls) Block Diagrams in Appendix A, Electrical Section.

The RTI F2 QD 1/20th scale model Control Strategy uses a two-tiered control system. The Tier 1 back-up system is the simplest, and is similar to the system used in our 1/50th scale model where the wave resistive force (generator torque) was directly proportional to Float angular velocity (RPM). In this system the multi-pole permanent magnet motor-generator is used as a 3 phase AC generator connected to 3 adjustable 0-100 ohm, 100 watt resistors. In the Tier 2 primary control system, the same motor-generator will be used as a motor with its torque digitally controlled during each wave cycle utilizing a digital torque “gain” preset prior to testing during each wave test condition. The torque gain during Float up-strokes (during oncoming wave crests) may be set to a different level (generally larger) than the torque gain setting for ensuing wave troughs (where Float is falling). While this system allows use of wave to wave adaptive control based on sensing individual oncoming wave heights and arrival times (using RTI pressure transducers or Carderock Wave Staffs), the short Wave Energy Prize design, build, and pre-testing schedule precluded implementation of an adaptive control system.  The RTI 1/20th model also has an Electro-Mechanical Brake (EMB) which can be used for “latching” at the 0 RPM start (wave trough) of each wave cycle and disengaged at either a preset time or torque (based on the average wave period or amplitude for that test set) (or based on the pre-measured oncoming wave if adaptive wave-to-wave control is used). Again the short WEP schedule did not provide time to pre-test the EMB for “Latching” control. Our high torque permanent magnet hub-motor type motor-generator with its 8:1 gearbox and our 4:67 chain drive does provide relatively high torque.

Additionally;

1. Regarding software control response to programed architecture and on-board and remote sensors, we do not plan a control response using Carderock’s optical tracking system. We would, of course, like to receive optical tracking video and data for future R&D use.

Our on-board sensors consist of our 2 submersible tension-compression Load Cells (one on each split and hinged Drive or Swing Arm) and 2 high resolution magnetic encoders (both on common drive axel).  Because the 2 Drive arms are connected to a common Drive Axle, Loads must be measured in both Cells and summed to get total torque. We have a back-up provision to unload either cell and use only one cell for total torque (if either cell fails) but it places higher stresses on the single loaded Drive Arm (and may result in Drive Arm structural failure, especially in the extreme wave cases).

2.  We do plan to add or subtract high density ballast weight discs to our Float (deck), and or Frame (Horizontal Drag Plate) between tests (using boat hook and/or Carderock punt). As described in Section 4.3 Device Properties and Dev ice Froude Scaling above, the Full Scale RTI F2 QD device can readily provide a wider range of Float and Frame weight changes than the 1/20th scale weight discs, by utilizing the Float and Frame seawater ballasting systems, the automated operation of which has been previously described.

If our electronic motor torque control system malfunctions at Carderock, we will use, as a back-up plan, our 3 resistor loads (3 resistors each rated at 100 watts and adjustable from 0 to 100 ohms) using our motor as a generator.

# Appendix E: Raw Data Channel list

| Data File: |  | The name of the data file where the data resides |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Name: |  | The name of the data channel in the data file - if data are in a matrix, this is the index (column number) of the data in the matrix |  |  |  |  |  |
| Channel Title: |  | The common named use to refer to the data in the cannel |  |  |  |  |  |
| Description: |  | Description of what is being measured/recorded |  |  |  |  |  |
| Unit: |  | The unit of the measurement as output by the DAS |  |  |  |  |  |
| Sensor |  | The name of the sensor. Please provide enough information so a reader can identify the specific type of sensor used |  |  |  |  |  |
| Sample Rate |  | The sample rate of the data record |  |  |  |  |  |
| Scaling and Conversion Calculations |  | Where the measured values will be used |  |  |  |  |  |
| Data File | Channel Name | Channel Title | Description | Unit | Sensor | Sample Rate | Scaling and Conversion Calculations |
| WEC Prize Test Sensors |  |  |  |  |  |  |  |
|  |  | Mooring Tension 1 |  |  |  | 50 Hz | See data file for conversions used during testing |
|  |  | Wave Height 1 |  |  |  | 50 Hz |  |
|  |  | Wave Height 2 |  |  |  | 50 Hz |  |
|  |  | Wave Height 3 |  |  |  | 50 Hz |  |
|  |  | Wave Height 4 |  |  |  | 50 Hz |  |
|  |  | Wave Height 5 |  |  |  | 50 Hz |  |
|  |  | Wave Height 6 |  |  |  | 50 Hz |  |
|  |  | Wave Height 7 |  |  |  | 50 Hz |  |
|  |  | Wave Height 8 |  |  |  | 50 Hz |  |
| Team Sensors |  |  |  |  |  |  |  |
|  |  | Water Pressure 1 | 6 PSI pressure transducer |  | PX180B-006GV | 1 kHz | Gain 600 |
|  |  | Water Pressure 2 | 6 PSI pressure transducer |  | PX180B-006GV | 1 kHz | Gain 600 |
|  |  | Mechanical Force Left | 1000 lbf load cell |  | WMC-1000 | 1 kHz | Gain 2.1636E+6 |
|  |  | Mechanical Force Right | 1000 lbf load cell |  | WMC-1000 | 1 kHz | Gain 2.1636E+6 |
|  |  | Auxiliary Sensor 1 |  |  |  | 1 kHz | N/A |
|  |  | Auxiliary Sensor 2 |  |  |  | 1 kHz | N/A |
|  |  | Auxiliary Sensor 3 |  |  |  | 1 kHz | N/A |
|  |  | Auxiliary Sensor 4 |  |  |  | 1 kHz | N/A |
|  |  | Magnetic Encoder Left | 8192 P/rev Quadratic Magnetic Encoder |  | BEI HHU9 | 1 kHz | Gain 0.00197 |
|  |  | Magnetic Encoder Right | 8192 R/rev Quadratic Magnetic Encoder |  | BEI HHU9 | 1 kHz | Gain 0.00197 |

# Appendix F: Checklists for spot checks, software operation, readiness verification, and “real time” data QA

Initial:

- Verify Drive Arms are in the desired center of travel at the still water condition (approx. 350)

- Verify and note base line Float Weights (20 lb. +/-) and Frame-Drag Plate Weight (50 lb. +/-)

Every run:

- Record water temp,

- Record wave test point

- Check for loose bolts where possible

- Verify integrity of optical markers

- Verify telemetry (loads, position, motion tracking) is recording

- Record any still water pre-load on Load Cells

## Spot Check Procedure

Prior to testing, the first spot check will be performed in the Carderock parking lot outside of the MASK facility. This can be done on the Thursday or Friday of Set-Up Week to save Test Week Time. The spot check will be performed to verify the accuracy and the scale of the dynamic and kinematic sides of wave power capture which use either of two (2) rotary encoders (mounted on both ends of a common Drive Axel) and both the port and starboard load cells, respectively.

The Spot Check procedure is performed with the F2 QD Frame Assembled from the Mid-Frame Vertical Spar section and above (including the PTO Housings, upper Vertical Spars, and upper Cross Truss beam but excluding the Lower Vertical Truss sections and Drag Plate Assembly) bolted to a Temporary Assembly Stand which holds the Frame and PTO Housings in their vertical position.

To spot check the 2 Load Cells (port and starboard side) mounted on their respective Drive Arms in both tension and compression, the Drive Arms are rigidly connected by their clamping couplings to the common Drive Axel and oriented exactly horizontally (using a Carderock provided level). The Arms are maintained in a horizontal position (placing the Load Cells in compression) using an RTI provided Drive Axel Locking Bar which prevents Drive Axel Rotation via contact with the Temporary Assembly Stand. Either multiple and varying combinations of RTI 2.5 lb. and 5.0 lb. Frame and Float Weights (about 100 lbs. total) or Carderock provided weights are hung from the outermost Drive Arm to Float Mounting holes on the Arms and the Load Cell readings are noted which are equal to the added weight times the Mounting hole to Drive Axel axis distance/Load Cell to Drive Axel axis distance (approx. 2.5:1). The Drive Axel is then rotated 1800 (still horizontal) and reloaded with weights on the Drive Arm placing the Load Cells in tension. While the Load Cells are rated at 1,000 lbs. maximum for robustness, the Cells should not see more than 150 lbs. in tension (gravity of 50 lb. Float with max 25 lb. Float Weights x 2.5 lever ratio over 2 Arms). Maximum compressive loads (from buoyant and kinetic wave forces on the Float) on the cells should be below 500 lbs. RTI would prefer not to spot check at more than the above maximum anticipated loads to avoid stripping the threads (which are only 1/4”-28) on these small (but expensive) Load Cells.

The BEI high resolution magnetic encoders can also be spot checked using the same Temporary Assembly Stand set-up described above. The Encoders are mounted to each end of the common Drive Axel and are therefore redundant. It is anticipated that one encoder signal will go directly to the Carderock DAQ and the other to the RTI DAQ. The Drive Axel Locking Bar described above, which is attached to the Drive Axel with its own clamp coupling, can be removed or loosened allowing the Drive Arms and Drive Axel to rotate through their anticipated maximum travel arc (350 from horizontal in Still water plus 350 on wave crests and minus 450 in wave troughs). With the Float detached either Drive Arm (both are rigidly connected to the Drive Axel) can be rotated to various angles (measured by a laser, bubble, or other level measure provided by Carderock) and the encoder readings noted and calibrated against the level measurements.

The spot check apparatus utilized in the Load Cell and Encoder spot check procedures described above utilize RTI’s Temporary Assembly Stand which consists of two (2) 2” x 4” x 6 ft. length wood timbers and a single 2” x 4” x 6 ft. aluminum cross tie beam which rest on a flat surface and are both bolted to the bottom flanges of the two (2) Mid-Frame Vertical Spar sections (short vertical Frame sections directly below the 2 PTO housings). This allows the F2 QD mid and upper Frame to remain in a vertical orientation without elevating it with the Lower Vertical Spar Frame sections (which would require ladder access for assembly, wiring, and spot checks). The Spot checks also require the Drive Axel Locking Bar (by RTI), gravity weights (up to 100 lbs. by RTI with additional weights by Carderock), a scale for pre-weighing weights (by Carderock), and a laser or bubble type level for encoder angle calibration (by Carderock). Readings of the Load Cells and Encoder signals can be by use of either RTI or Carderock/DA DAQ units (or both).

The following two spot checks will be performed:

- Load cell performance by incrementally increasing and decreasing weight added to the float. Data will be collected for an interval of 60 seconds for each individual load. At least 5 increments will be needed spanning the full anticipated loads on the load cell. This should be repeated twice.

- Rotary encoder performance covering the full range of the encoder with increments of 30 degrees. Data will be collected for an interval of 60 seconds for each individual load. Ideally, finer increments should be used within the range of expected motions. This should be repeated twice. Because the 2 encoders are directly on the Drive Axel which is rigidly connected to the Float by the 2 Drive Arms, RTI does not anticipate a travel arc of more than 800 but the Drive Axel with Drive Arms attached can be rotated through a complete 3600 if desired.

| Spot Check 1 – Load cells |  |  |
| --- | --- | --- |
| Calibration weight applied (lbf) | Load cell reading Load cell 1 (mv/V) | Load cell reading Load cell 2 (mv/V) |

| Spot Check 2 – Rotary Encoder |  |  |
| --- | --- | --- |
| Directly Measured Angle (units) | Rotary encoder reading Rotary Encoder 1 (units) | Rotary encoder reading Rotary Encoder 2 (units) |

For every run, the following check will be performed to ensure the WEC is ready for testing

- Check for loose bolts, appendages, and connectors

- Check and Note Load Cell pre-loads (tension or compression) in still water

- Check for changes in still water Load Cell pre-loads which may indicate slippage between Drive Arms and Drive Axel. Tighten Drive Arm Clamping Couplings if necessary.

- Water level of both device Frame and Float

- Add or subtract Float Weight and/or Frame-Drag Plate Weight for different Wave Conditions to simulate Full Scale adjustable Seawater Ballasting.

- Verify integrity of optical markers

- Verify telemetry (loads, position, motion tracking) is recording

- Verify umbilical is free of tangles and twists and confirm that umbilical cables are not impeding F2 QD model’s self-orienting weather vanning properties.

# Appendix G: Team provided sensors – specifications and calibrations

Port Drive Arm Load Cell

Starboard Drive Arm Load Cell

# Appendix H: Data analysis details

The following time series will be plotted for each run and be available for viewing between runs (if time permits)

| Variable | Definition | Reference Formula | Relevant Runs |
| --- | --- | --- | --- |
| Displacement – Inertial Frame | X is defined relative to the 0 deg wave heading, Z is upward and Y completes the right hand rule | X= X,Y | All |
| Mooring Tension for each line | The instantaneous value of the mooring tension for line j | Fj | All |
| Kinematic Power | Kinematic Side of Power for PTO j | kinj | All |
| Dynamic Power | Dynamic Side of Power for PTO j | dynj | All |
| Absorbed Power | Absorbed power for PTO j | Pj | All |

The following variables will be calculated for each run and be available for viewing between runs (if time permits)

| Variable | Definition | Reference Formula | Relevant Runs |
| --- | --- | --- | --- |
| Wave PSD | Spectral density of the water surface elevation | SHf | All |
| Significant Wave Height | Measured significant wave height | Hm0=4m0 where mi=k=f0fNfkiSkf∆fk | All |
| Omni-Directional Wave Energy Flux | Omni-Directional Wave Energy Flux | J= ρgk=f0fNSkfCgk∆fk Cgk= 12Cpk1+2kkhsinh2kkh Cpk= gkktanhkkh | All |
| Wave Energy Period | Wave Energy Period | Te=m-1m0 | All |
| Horizontal Displacement | Horizontal displacement of the WEC from its at rest position | Υ=X2+Y2 | All |
| Mean | The mean value of the mooring tension for line j | Fj=1Ni=1NFji | All |
| Standard Deviation | The standard deviation of the mooring tension for each mooring line | σFq=1N-1i=1MFji-Fj2 | All |
| Max | The maximum value of the mooring tension of all mooring lines | maxFj | All |
| Min | The minimum value of the mooring of all mooring lines | minFj | All |
| Mean | The mean value of the kinematic side of power for PTO j | kinj=1Ni=1Nkinji | All |
| Standard Deviation | The standard deviation of the kinematic side of power for PTO j | σkinj=1N-1i=1Mkinji-kinj2 | All |
| Max | The maximum value of the kinematic side of power | maxkinj | All |
| Min | The minimum of the kinematic side of power for PTO j | minkinj | All |
| Kinematic spectral density | Spectral density of the kinematic side of power for PTO j | Skinjf | All |
| Mean | The mean value of the dynamic side of power for PTO j | dynj=1Ni=1Ndynji | All |
| Standard Deviation | The standard deviation of the dynamic side of power | σdynj=1N-1i=1Mdynji-dynj2 | All |
| Max | The maximum value of the dynamic side of power | maxdynj | All |
| Min | The minimum of the dynamic side of power | mindynj | All |
| dynamic spectral density | spectral density of the dynamic side of power, one for each power conversion chain of the WEC | Sdynjf | All |
| Mean | The mean value of the power | Pj=1Ni=1NPji | All |
| Standard Deviation | The standard deviation of the power | σPj=1N-1i=1MPji-Pj2 | All |
| Max | The maximum value of the power | maxPj | All |
| Min | The minimum of the power | minPj | All |
| Absorbed power spectral density | spectral density of the absorbed power, one for each power conversion chain of the WEC | SPjf | All |

This page intentionally left blank