Wave Energy Prize

Waveswing America Test Plan

Version 5, August 1st, 2016

This page intentionally left blank

# Content

Content	3

List of Tables	5

List of Figures	6

1	Introduction	8

2	Test Objective	9

3	Test Facility	10

3.1	Wave Maker	12

3.2	MASK Orientation	13

4	Scaled Model Description	15

4.1	Device description	15

4.1.1	Full-scale device	15

4.1.2	Model-scale device	16

4.2	Power Take-Off description	20

4.3	Device properties	21

4.4	Froude scaling	24

4.5	Other scaling or measures	24

4.6	Control strategy	24

4.6.1	Feedback damping control	26

5	Test Matrix and Schedule	29

5.1	Test matrix	29

5.2	Test schedule	29

6	Experimental Set Up and Methods	31

6.1	Mooring	31

6.2	Instrumentation	35

6.3	Motion tracking	37

7	Data Processing and Analysis	38

7.1	Data quality assurance and on-site processing	38

7.2	Data analysis	38

7.2.1	“Real Time” Data QA	40

7.2.2	Settling Interval and Time between Test Data QA	40

8	Data Management	42

9	References	43

Appendix A: Device electrical and mechanical drawings	44

Appendix B: Device Froude scaling	48

10	Provisions for dealing with air-compressibility issue s and spring effects at 1:20 scale	50

(refer to AWS File Note FN 16-001)	50

10.1	Operation of the air-springs within the Archimedes Waveswing	50

10.2	Full-scale spring response (per 15-004)	50

10.3	Model-scale spring response for 1:20 model	51

10.4	Effect of additional floater mass at model scale	53

10.5	Conclusion	53

Appendix C: Detailed description of control strategy	54

Appendix D: Raw Data Channel list	55

Appendix E: Checklists for spot checks, software operation, readiness verification, and “real time” data QA	56

Spot Check Procedure	56

Appendix F: Team provided sensors – specifications and calibrations	58

Appendix G: Data analysis details	65

# List of Tables

Table 1. Test waves	29

Table 2. Testing schedule	29

Table 3:  Spring forces at full scale and 1:20 scale	52

# List of Figures

Figure 1. General Schematic of bridge and MASK basin.	11

Figure 2. General view of new segmented wavemaker in MASK Wavemaking Facility. Paddles are highlighted in red and the control cabinets are highlighted in bright blue.	12

Figure 3. MASK carriage shown below the bridge at the center of the bridge.	12

Figure 4. General wavemaker characteristics and design	13

Figure 5. MASK reference orientation, note that the orientation here is different than that in Figures 1 and 3.	14

Figure 6:  Concept design of full-scale Waveswing	15

Figure 7: Schematic layout for 1:20 Scale Model	17

Figure 8:  Engineering drawing of the Waveswing 1:20 model absorber head	18

Figure 9:  Exploded parts view of the Waveswing 1:20 model absorber head.	19

Figure 10: Sectional view of floater and Silo showing PTO mounting and load cells	20

Figure 11:  Waveswing control states	25

Figure 12:  Waveswing 1:20 scale control panel HMI screen	26

Figure 13:  Waveswing anchor frame and air reservoir	31

Figure 14:  Engineering drawing of anchor frame with reservoir removed	32

Figure 15:  Anchor frame mass and displacement table	32

Figure 16:  Kinematic sensor load cell calibration in tension	36

Figure 17. Data flow and processing steps	39

Figure 18 - Full-scale device desired spring response (after 15-004 Figure 2)	51

Figure 19:  Comparison between scaled spring curve and actual 1:20 model spring curve	52

This page intentionally left blank

# Introduction

The Department of Energy (DOE) launched the Wave Energy Prize (WEPrize) Competition as a mechanism to stimulate the development of new wave energy converter devices that have the prospect of becoming commercially competitive in the long run.  In the Final stage of the competition, nine teams will test their 1/20th scale devices at the US Naval Surface Warfare Center Carderock Division (NSWCCD) Maneuvering and Seakeeping Basin (MASK) in West Bethesda, MD.   Each contestant will prepare their device for one week and then test their device for one week at the MASK basin in Summer/Fall 2016.  This testing program will measure the performance of each device tested to determine the WEPrize winners.

The purposes of the Team Test Plan are to:

- Plan and document the 1/20th scale device testing at the Carderock MASK basin;

- Document the test article, setup and methodology, sensor and instrumentation, mooring, electronics, wiring, and data flow and quality assurance;

- Communicate the testing between the Finalist team, Carderock, Data Analyst (DA) and the Prize Administration Team (PAT);

- Facilitate reviews that will help to ensure all aspects (risk, safety, testing procedures, etc.) have been properly considered;

- Provide a systematic guide to setting up, executing and decommissioning the experiment.

The team test plan is a WEPrize required document and will be owned/managed by the Carderock Test Leads and DAs, and is intended to be a “living document” that will evolve continuously prior to the MASK basin testing.

# Test Objective

The top level objective of the 1/20th scale device testing is to obtain the necessary measurements required for determining Average Climate Capture Width per Characteristic Capital Expenditure (ACE) and the Hydrodynamic Performance Quality (HPQ), key metrics for determining the WEPrize winners [1].

# Test Facility

All testing will be conducted in the Maneuvering and Seakeeping basin (MASK) at Carderock Division, Naval Surface Warfare Center located in Bethesda, Maryland.  The MASK is an indoor basin having an overall length of 360 feet, a width of 240 feet and a depth of 20 feet except for a 35-foot deep trench that is 50 feet wide and parallel to the long side of the basin.  The basin is spanned by a 376-foot bridge supported on a rail system that permits the bridge to transverse to the center of the basin width as well as to rotate up to 45 degrees from the centerline as seen in Figure 1.  Figure 1 does not include the physical update of this wave maker system, but a drawing of the new paddle layout can be seen in Figure 2.  The MASK Carriage is suspended beneath the bridge and can travel along the rails by the rollers and drive system.  There is an arresting gear to prevent the carriage from hitting the end stops and this limits the travel along the bridge.  The carriage has 6’ x 10’ moon bay in the center which allows for models and instrumentation to be mounted.  A photo of the carriage is shown in Figure 3.  Along the two ends opposite of the wave makers are beaches with a 12 degree slope. The beaches are constructed of 7 layers of concrete sections and are effective in mitigating the mass flux of water back into the tank during wave generation.  The hydrodynamic properties of the beaches can be found in [2].

Figure 1. General Schematic of bridge and MASK basin.

Figure 2. General view of new segmented wavemaker in MASK Wavemaking Facility. Paddles are highlighted in red and the control cabinets are highlighted in bright blue.

Figure 3. MASK carriage shown below the bridge at the center of the bridge.

## Wave Maker

The new wave maker is rendered with respect to its general install position in Figure 2.  The wave maker system consists of 216 paddles.  There are 108 paddles along the North edge of the basin, 60 paddles in a ninety-degree arc, and 48 paddles along the West edge of the basin.  The paddles are grouped in sets of eight paddles per control cabinet.  The 27 control cabinets are then joined via three marshaling cabinets, and ultimately the marshaling cabinets are connected to the main control station at the second floor of the MASK control room.  The cabinets and control room are generally illustrated in Figure 2.

A more detailed view of the wave maker paddles is provided in Figure 4.  The paddles have a hinge depth of 2.5 m (8.2 ft.) and a pitch (centerline to centerline spacing) of 0.658 m (25.9 in.).  The wave maker system is a dry back, force feedback system.  The paddles are moved using hydrostatic compensation with air tanks and bellows and with sectors attached to the wave makers with an A-frame type structure.  The sector has a timing belt attached which runs on the topside of the sector.  The timing belt runs through a pulley box powered with an encoder controlled motor.  The motor is used to control the real-time quick motions of the paddle.  The force feedback of the paddle is provided via a force transducer mounted at the bellows and sector interface to the paddle.

The wave maker is controlled via runtime software located on the main control computer using Edinburgh Designs Limited (EDL) software. The software allows entering specific regular wave conditions or it can be programmed to generate irregular seas via the input of “experiments files”.

Figure 4. General wavemaker characteristics and design

## MASK Orientation

With respect to the MASK basin, the reference frame is illustrated in Figure 5.  Its operational origin is located at the interior intersection of the northwest and northeast walls and vertically at the nominal 20 ft. water level.  The positive x-axis is aligned along the shorter northwest wall and the positive y-axis along the longer northeast wall.  Waves propagating parallel with the x-axis (toward the long beach) are defined as having a mean wave direction, β₀, of zero degrees and waves propagating parallel with the y-axis as 90 degrees.  This convention defines the wave direction as the direction the waves are traveling toward.

Figure 5. MASK reference orientation, note that the orientation here is different than that in Figures 1 and 3.

# Scaled Model Description

## Device description

### Full-scale device

The Archimedes Waveswing™ is a submerged self-reacting point absorber driven by differential pressures caused by incident waves.  In simple form the device is a submerged telescopic structure with a lower part secured by a mooring and the upper part free to move vertically.  The structural elements of the full scale device are shown in drawing Waveswing_Q2_01_20151130.pdf sheet 1, and are as follows:

- The Floater:  The moving part of the device, providing the wave absorbing surface;

- The Silo:  The fixed part of the ‘telescope’ and contains the PTO and other equipment;

- The Leg:  A tension strut to connect the Silo to the anchor;

- The Anchor:  A gravity base or suction-embedded plate type anchor designed for the site conditions.

Figure 6:  Concept design of full-scale Waveswing

The Floater and the Silo together form two gas chambers.  The inner chamber housing the PTO contains a partial vacuum whilst the outer chamber contains air at a pressure equal to the hydrostatic pressure of the free-surface in the gap between the floater and the Silo.  The Gap volume is connected to a reserve volume within the Leg by means of ducts which also form part of the bearing structure.  The combined forces acting on the Floater due to the two pressure chambers are arranged to balance the external hydrostatic forces at equilibrium.  As the external pressure increases due to a passing wave crest, the Floater is driven downwards, thus reducing the down-force caused by the partial vacuum and increasing the up-force due to compression of the air in the gap.  The reverse happens under a wave trough.  Through careful balancing of the gap and silo volumes, a ‘soft’ and tune-able spring is produced.  Tuning is achieved by varying the volume of water within the leg lower part of the Silo, thus varying in the compression ratio of the Gap volume.  For shut-down or storm survival, water is released from the leg Silo, thus de-pressurizing the Gap volume and causing the Floater to come to rest at bottom of stroke.

The device will typically be deployed in water depths of greater than 50m and provisional full-scale dimensions are a Floater diameter of 4.0m and a stroke of 4.0m.  The minimum submergence of the device at ‘top-dead-center’ of stroke is 4.0m.

### Model-scale device

At model scale it is not possible to configure all of the gas chambers as per the full-scale design, furthermore, compressibility of the internal gas does not scale and therefore it is necessary to provide an additional air reservoir to provide sufficient compression of the internal air.  This arrangement was used successfully for the 1:50 scale model tests and the proposed arrangement for the 1:20 tests at Carderock is shown in Figure 7

Waveswing

Pull-down and load-cell

Hollow leg

Air reservoir

Anchor frame

Figure 7: Schematic layout for 1:20 Scale Model

The engineering drawings for the device are shown in Figure 8 and Figure 9.  It should be noted that due to model scale constraints, principally the size of sensors and the PTO unit which do not scale properly, and the need to ensure free movement of air to the hollow leg, the silo portion of the device is proportionally longer than will be necessary at full scale.  This is not however expected to impact performance.

Figure 8:  Engineering drawing of the Waveswing 1:20 model absorber head

Figure 9:  Exploded parts view of the Waveswing 1:20 model absorber head.

Larger versions of these drawings are provided in Appendix [HOLD] and provisional dimensions are provided in section 4.3.

## Power Take-Off description

The power take-off will be a single direct-drive permanent magnet linear motor manufactured by LinMot Inc type PS01-37X120-C with high-clearance slider.

The moving magnet stack (slider) will be attached to the Floater via a flexible coupling whilst the motor armature is mounted within the silo.  The motor force is controllable via an analogue signal provided to the motor drive which allows real-time dynamic force control including both spring and damping terms.  Load cells are provided between the slider and the floater, thus measuring the total direct force applied by the floater to the PTO as shown in Figure 10.

Floater

Silo body

Load cells

Motor slider

Motor armature

Figure 10: Sectional view of floater and Silo showing PTO mounting and load cells

The motor incorporates high-precision position measurement via a built-in encoder from which velocity can be derived in real-time.  The kinematic component of power will be measured via load-cells mounted between the motor slider and the floater top.

The linear motor is controlled by a LinMot B1100-GP drive unit which is mounted within the device controls cabinet

The velocity response of the Floater to an incident wave (and hence resistance force) provided by the PTO will be controlled using a dynamic control algorithm implemented within a real-time target  National Instruments cRIO high-speed controller.  The algorithm will provide compensation for floater inertia (as the model-scale mass is unavoidably high), together with spring offset force balancing and adjust damping according to floater position, velocity and acceleration in order to maximize power capture whilst avoiding end-stop events.

## Device properties

Table 1 Critical Dimensions

|  | Measurement |  |  | Full-scale | 1:20 scale (target) | 1:20 scale (Actual) | Comments |  |  | Verified at Carderock |  | Measured by Team |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer Diameter of Floater |  |  | 4.0m | 0.200 | 0.198 |  |  |  |  |  |  |
|  | Height of Floater (Cap A) |  |  | 9.4m | 0.55 | 0.55 |  |  |  |  |  |  |
|  | Outer Diameter of Silo |  |  | 3.2m | 0.160 | 0.160 |  |  |  |  |  |  |
|  | Height of Silo |  |  | 8.4m | 0. 470 | 0.420 |  |  |  |  |  |  |
|  | Outer Diameter of Leg |  |  | 1.2m | 0.06 | 0.060 |  |  |  |  |  |  |
|  | Stroke Length |  |  | 4.5m | 0.225 | 0.204 |  |  |  |  |  |  |
| Overall Mass (kg) |  |  |  |  | 6.2kg |  |  |  |  |  |  |  |
| Overall Displacement with float at TDC (m3) |  |  |  |  | 0.0218m3 |  |  |  |  |  |  |  |
| Overall Displacement with float at BDC (m3) |  |  |  |  | 0.0172 m3 |  |  |  |  |  |  |  |
| Overall Center of gravity with float at TDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall Center of gravity with float at BDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall Center of buoyancy with float at TDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall Center of buoyancy with float at BDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance from Floater crown to still water surface at TDC |  |  |  |  | 0.400 |  |  |  |  |  |  |  |
| Floater and Silo mass |  |  |  |  |  |  |  |  |  |  |  |  |
| Floater and Silo Displacement with float at TDC (m3) |  |  |  |  | 0.017397m³ |  |  |  |  |  |  |  |
| Floater and Silo Displacement with float at BDC (m3) |  |  |  |  | 0.011645m³ |  |  |  |  |  |  |  |
| Floater and Silo Center of gravity with float at TDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Floater and Silo Center of gravity with float at BDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Floater and Silo Center of buoyancy with float at TDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Floater and Silo Center of buoyancy with float at BDC (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Moment of inertia of Floater and Silo at TDC |  | Pitch (kgm2) |  |  |  |  |  |  |  |  |  |  |
|  |  | roll (kgm2) |  |  |  |  |  |  |  |  |  |  |
|  |  | yaw (kgm2) |  |  |  |  |  |  |  |  |  |  |
| Moment of inertia of Floater and Silo at BDC |  |  | Pitch (kgm2) |  |  |  |  |  |  |  |  |  |
|  |  |  | roll (kgm2) |  |  |  |  |  |  |  |  |  |
|  |  |  | yaw (kgm2) |  |  |  |  |  |  |  |  |  |
| Leg mass (kg) |  |  |  |  |  |  |  |  |  |  |  |  |
| Leg Displacement (m3) |  |  |  |  |  |  |  |  |  |  |  |  |
| Leg Center of gravity (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Leg Center of buoyancy (m) from SWL |  |  |  |  |  |  |  |  |  |  |  |  |
| Moment of inertia of Leg |  |  | Pitch (kgm2) |  |  |  |  |  |  |  |  |  |
|  |  |  | roll (kgm2) |  |  |  |  |  |  |  |  |  |
|  |  |  | yaw (kgm2) |  |  |  |  |  |  |  |  |  |

## Froude scaling

Device linear dimensions will be scaled according to Froude scaling laws.

The device mass will be unavoidably high as compared with Froude scaled values due to limitations on material thicknesses and strength at 1:20 scale.  The scaled mass of the moving floater at model scale is almost double the target value which will have a negative impact on device performance.  This is however unavoidable.

The air and vacuum spring systems will be replaced with a linear mechanical spring and an enlarged air reservoir all as described in the Team’s submission regarding Froude scaling – see File Note entitled Waveswing America – Waveswing™ Froude scaling for 1:50 scale test, FN 15-004, 21 August 2015.  File note FN 16-001 provides full details of the scaling approach for the 1:20 scale model and the contents are reproduced in Appendix C of this test plan.

PLEASE REFER TO APPENDIX C FOR FULL DETAILS OF AIR-SPRING SCALING PROVISIONS.

It should be noted that (a) the final full-scale device spring constants will be variable by changing the device operating parameters on a sea-state by sea-state basis to suite conditions and to mimic this, (b) the model spring constant can be adjusted by introducing additional control terms in the linear motor controller and by varying the volume of air contained in the compensation reservoir.

Accordingly, the Team believes that an adequate and representative scaling of the air/vacuum spring will be achieved such that the performance of the scale model will be an accurate scaled representation of the full scale device.  Refer to Figure 2 in Appendix C of this Test Plan.

Table 2 General Froude Scaling

| Dimensions | Description of Dimension | Full Scale Value | 1:20th Scale Value |
| --- | --- | --- | --- |
| Length | Floater diameter | 4.0m | 200mm |
|  | Silo diameter | 3.2m | 160mm |
|  | Stroke | 4.5m | 225mm |
|  | Floater length | 8.0m | 400mm |
|  | Silo length | 8.4m | 420mm |
|  | Stub-leg diameter | 1.2m | 60mm |

## Other scaling or measures

See above (4.4) and details provided in Appendix C re addition of air reservoir.

## Control strategy

The 1:20 scale Waveswing is a fully submerged and remotely operated system which must be controlled properly in order to ensure integrity.  This is achieved using a control system based on a  National Instruments cRIO unit with appropriate sensors and I/O.  The overall control strategy for the Waveswing is semi-automatic based on a number of operational states as follows:

Table 3 Control States

| State | Name | Description / notes |
| --- | --- | --- |
| 1 | Stopped | Device in safe shut-down mode with floater at lower position and air de-pressurized slightly to ensure that floater-silo seal is maintained. Controls remain live to monitor air pressure and reservoir water level. |
| 2 | Startup under manual control | Operator runs device through start-up routine including checking air pressures, moving floater to mid-stroke equilibrium position, checking force-control capability and other system checks. |
| 3 | Auto-run Mode 1 – force control | Device in automatic mode and will respond to wave inputs with PTO damping controlled by force-feedback control |
| 4 | Auto-run Mode 2 – linear damping control | Device in automatic mode and will respond to wave inputs with PTO damping set to single linear value |
| 5 | Manual control | Device in fully manual control with floater position controlled through HMI |

Figure 11:  Waveswing control states

During performance testing it is intended that the device is operated in State 3 with State 4 being available as a fall-back should the controls algorithms prove unreliable.  State 5 is used only for positioning the floater and carrying out forced oscillation tests (not part of Carderock WEP tests).

The only adaptive strategy available is the adjustment of reservoir water level (and hence compressible air volume) so as to adjust the resonant response of the device to maximize energy extraction from a particular sea-state.  This will be achieved by operation of the reservoir bilge pump or by introducing water to the system via the pump line.  Pre-testing has shown that this adaptive strategy is not required, with an equivalent result being obtained through adjustment of the linear spring compensation via the PTO control.  Hence this remains optional at this time.

With regards to the PTO control, the active mode is described as follows:

### Feedback damping control

The feedback damping control uses a combination of strategies as follows:

- Primary control is by means of an algorithm similar to that implemented in absorbing wave-makers which uses dynamic force and velocity feedback in order to produce a damping response which matches the computed radiation damping of the device.  This allows optimum power absorption at any time without need for future knowledge of the incoming wave;

- Secondary control (necessary in larger waves to avoid end-stop collisions) uses an algorithm which uses the dynamic force, position and velocity feedback to calculate the PTO response required [REDACTED] to avoid end-stops, whilst avoiding over-damping;

The outputs of the algorithms will be summed to provide the overall PTO response required.  Accordingly, the PTO may control to optimal damping in the mid-stroke region with energy (hence stroke) limiting kicking in towards the ends of the stroke.

The block diagram for the LabView software deployed on the cRIO is provided in Appendix [HOLD] and a screen-shot of the front panel is provided in figure [HOLD] below

Figure 12:  Waveswing 1:20 scale control panel HMI screen

The parameters adjustable via the front panel during testing are as follows:

Table 4 Control Parameters

| Parameter | Default value | Purpose |
| --- | --- | --- |
| Frequency (hz) | n/a | Used to set frequency in forced oscillation tests |
| Amplitude (mm) | n/a | Used to set amplitude in forced oscillation tests |
| L-spring | Sea-state dependent | Provides positive or negative linear spring as mechanical spring effect is cancelled by the PTO due to position of the load cell |
| Acc Gain | 0.03 | Provides positive or negative acceleration-dependent force (i.e. mass) to compensate for excess mass at model scale |
| I Gain | 0.01 | Integral gain of the main PI force control loop |
| P Gain | 0.01 | Proportional gain of the main PI force control loop |
| Res offset | 0 | Device submergence depth used to calculate free-board (distance from annular water surface to top of silo) which is used to provide warning of potential inundation of silo |
| Add to offset | 0 | Addition to the force offset value. Force offset is used to balance floater buoyancy to mass and mechanical spring. Necessary as PTO cancels mechanical spring force due to load cell position. Additional offset used for fine adjustment to variations in internal air pressure |
| Base (kN/m/sec) | Sea-state dependent | Used in simple control mode to set the linear damping term |
| Lin zone (%) | Sea-state dependent | Used in simple control mode to set the % stroke over which linear damping is used |
| Non-lin peak | Sea-state dependent | Used in simple control mode to set the peak non-linear damping force in the end (i.e. non-linear) zone |
| AWS Spring | Sea-state dependent | Sets the total air and mechanical spring in the system. [REDACTED] |
| HS spring | 0 | Hydrostatic spring. Used in total energy calculation as per AWS spring |
| Mass | 6kg | Used in [REDACTED] stroke limitation algorithm |
| Added mass | 2kg | Hydrodynamic added mass used in [REDACTED] stroke limitation algorithm |
| Optimum damping | Sea-state dependent | Damping applied when stroke limits not reached |
| Max stroke | 0.22m | Sets max stroke used [REDACTED] stroke limitation algorithm |
| Stroke Dx | 0.05 | Distance increment for calculation of predicted stroke over-run in stroke limitation |
| Stroke Dt | 0.01 | Time increment for calculation of predicted stroke over-run in stroke limitation |
| Stroke K | Sea-state dependent | Gain for non-linear stroke limitation algorithm |
| Sample time | Varies | Total sample time for log file |
| File name | Varies | Name of log file |
| Threshold | 5m Bar | Sets air control dead-band threshold |
| Air on | 0.5 sec | Sets time of air charge / vent solenoid pulse |
| Air wait | 10 sec | Sets dwell time between successive solenoid pulses |
| Gauge scale | Varies | Scaling factor for capacitive gauge which measures water elevation in floater-silo gap |
| Gauge offset | Varies | Offset value for capacitive gauge which measures water elevation in floater-silo gap |
| E-zone | 10mm | Stroke end-zone for bump-stop protection. Over-rides all other force control outputs to ensure no mechanical impact |
| E-spring | 10 N/mm | Spring rate applied in end-zone to prevent mechanical impact |

# Test Matrix and Schedule

## Test matrix

The incident wave conditions for the 1/20th scale experiments at NSWC Carderock’s MASK are shown in Table 1.  Carderock will perform wave environment calibration in Summer 2016. The result of this calibration is shown in Appendix E.

Table 5. Test waves

| Type | Number | TP [s] | HS [m] | γ (gamma) | Direction | Spreading |
| --- | --- | --- | --- | --- | --- | --- |
| IWS (JONSWAP) | 1 | 1.63 | 0.117 | 1.0 | 10.0 | ∞ |
|  | 2 | 2.20 | 0.132 | 1.0 | 0.0 | ∞ |
|  | 3 | 2.58 | 0.268 | 1.0 | -70.0 | ∞ |
|  | 4 | 2.84 | 0.103 | 1.0 | -10.0 | ∞ |
|  | 5 | 3.41 | 0.292 | 1.0 | 0.0 | ∞ |
|  | 6 | 3.69 | 0.163 | 1.0 | 0.0 | ∞ |
| LIWS (JONSWAP) | 7 | 3.11 | 0.395 | 3.3 | -30.0 | 3.0 |
|  | 8 | 2.50 | 0.460 | 3.3 | -70.0 | 7.0 |
| RWS (4-parameter JONSWAP) | 9 | 3.22 | 0.076 | 2.0 | -70.0 | 7.0 |
|  |  | 1.61 | 0.108 | 2.0 | 0 | 10 |
|  | 10 | 3.32 | 0.079 | 2.0 | -70.0 | 7.0 |
|  |  | 1.93 | 0.065 | 2.0 | -10 | 10 |

## Test schedule

Table 6. Testing schedule

| Date/Time | Event |  |
| --- | --- | --- |
| Monday | MASK installation and work-in |  |
| 7:00 | Morning Huddle |  |
| 7:10 | Contestants will be moving their device from the assembly area to the installation area, installation and verifying operation |  |
| Tuesday | Continued installation and work-in |  |
| 7:00 | Morning Huddle |  |
| 7:10 | Contestants continue moving their device from the assembly area to the installation area, installation and verifying operation |  |
| 2:00 | Readiness verification |  |
| 3:00 | Baseline 1 run | IWS Wave 2 |
| 4:00 | Baseline 2 run | IWS Wave 5 |
| 4:20 | Contestants pack up for evening |  |
| Wednesday | Full Test Day |  |
| 7:00 | Morning Huddle |  |
| 7:15 | Contestants set up for testing and perform pre-test checks |  |
| 8:00 | Run 1 (Baseline 1) | IWS Wave 2 |
| 9:00 | Run 2 (Baseline 2) | IWS Wave 5 |
| 10:00 | Run 3 | IWS Wave 1 |
| 11:00 | Run 4 | IWS Wave 3 |
| 12:00 | Lunch |  |
| 1:00 | Check Run 1 (Baseline 1) | IWS Wave 2 |
| 2:00 | Run 5 | IWS Wave 4 |
| 3:00 | Run 6 | IWS Wave 6 |
| 4:00 | Check Run 2 (Baseline 2) | IWS Wave 5 |
| 5:00 | Contestants pack up for evening (also a 30 minute buffer) |  |
| Thursday |  |  |
| 7:00 | Morning Huddle |  |
| 7:15 | Contestants set up for testing and perform pre-test checks |  |
| 8:00 | Check Run 3 (Baseline 1) | IWS Wave 2 |
| 9:00 | Run 7 | RWS Wave 1 |
| 10:00 | Run 8 | RWS Wave 2 |
| 11:00 | Run 9 | LIWS Wave 1 |
| 12:00 | Lunch |  |
| 1:00 | Run 10 | LIWS Wave 1 |
| 2:00 | Check Run 4 (Baseline 2) | IWS Wave 5 |
| 3:00 | Backup Run 1/ Contestant Testing | TBD |
| 4:00 | Backup Run 2/ Contestant Testing | TBD |
| 5:00 | Perform final daily data QA checks and test reporting (may start earlier if testing permits) |  |
| Friday |  |  |
| 7:00 | Morning Huddle |  |
| 7:15 | Contestants set up for testing and perform pre-test checks |  |
| 8:00 | Backup Run 3/ Contestant Testing | TBD |
| 9:00 | Backup Run 4/ Contestant Testing | TBD |
| 10:00 | Backup Run 5/ Contestant Testing | TBD |
| 11:00 | Backup Run 6/ Contestant Testing | TBD |
| 12:00 | Lunch |  |
| 1:00 | Contestants pack up for shipping |  |

# Experimental Set Up and Methods

## Mooring

The Waveswing model is secured using a single anchor frame which also mounts the air reservoir and pull-down point for Waveswing model.  This is shown in Figure 13 below:

Figure 13:  Waveswing anchor frame and air reservoir

The complete assembly weighs [403]kg in air and has a total displacement of [360]kg.  Accordingly, it will be necessary to add a further [57]kg of ballast weight to the frame in order to ensure that it performs correctly in positioning the device.  The trays to either side of the base of the frame are designed to accept steel ballast billets supplied by Carderock.

An engineering drawing of the frame is shown in Figure 14.  [Note an A3 drawing can be provided as an appendix if required].

Figure 14:  Engineering drawing of anchor frame with reservoir removed

The frame should be lifted using the corner lift points and slings at least 4.0m long to ensure clearance from the frame and appurtenances.  The frame will be equipped with castors for movement on land.  These will be removed before deployment.

The mass & displacement table is shown in Figure 15 and the proposed deployment sequence is presented below.  The weight in water of the ballasted frame will be 100kg.

Table 7 Anchor frame mass and displacement table

| Component | Mass [kg] | Volume [litre] |
| --- | --- | --- |
| Frame | 212.0 | 27.1 |
| Pull bracket | 15.0 | 1.0 |
| Enerpac cylinder | 14.1 | 2.3 |
| Pulleys & shackles | 2.0 | 0.2 |
| Load cell | 5.0 | 0.8 |
| Lifting shackles | 4.0 | 0.5 |
| Reservoir | 150.0 | 321.0 |
| Tubes, etc | 1.0 | 6.0 |
| Additional ballast on reservoir | 50.0 | 7.1 |
| Totals | 403.1 | 358.9 |

|  | Step 1 – initial positioning - Assembled base-frame and reservoir wheeled to basin side on castors and positioned below runway-beam crane - Base loaded with additional ballast - Lifting slings attached and umbilicals flaked and prepared for lift |
| --- | --- |
|  | Step 2 – dockside preparation - Frame lifted by overhead crane and castors removed from underside of frame. Will require temporary dunnage to support frame for safety whilst persons underneath - Final check of slings and umbilicals prior to lifting into water |
|  | Step 3 – initial submergence - Frame lifted into water and lowered to tank floor - Umbilicals handled to keep above surface (in punt) - Reservoir raise / lower mechanism tested whilst assembly still on crane |
|  | Step 4 – transfer to lift barge - Lift barge brought to work area and straddles crane lift-point - Barge capstan line led through lift ring / shackle whilst frame load on tank bottom, then tensioned to take load - Crane hook removed leaving frame suspended below lift barge |
|  | Step 5 – lower & tow - Leg extension fitted to keep clear of surface and frame lowered close to tank floor - Barge towed to final test site location - (Note – sequence of tow / lowering may be reversed if considered necessary) |
|  | Step 6 – final position & checks - Frame lowered to final position whilst further leg extension added - Reservoir raise / lower mechanism tested again prior to removal of lift rig - Lift slings pulled clear of frame and laid out on tank floor. Capstan line removed from barge and led to gantry and secured. - (Alternative, tag line led through sling shackle to allow recovery) |
|  | Step 7 – attach Waveswing - Reservoir released to fully raised position - [If required, Carderock load cell checked for offset reading due to reservoir buoyancy at this stage] - Waveswing model attached to top of leg (using punt for access, not shown) - Umbilicals checked ready for final pull-down |
| Umbilicals | Step 8 – final pull-down - Reservoir retracted to position Waveswing model at working depth - Umbilical cables laid out clear of frame and taken to gantry above - Device commissioning begins |

Figure 15. Deployment Sequency

## Instrumentation

The sensors included within the Waveswing device are listed in [Table HOLD] below:

Table 8. Sensors

| ID | Function | Make | Type | Serial No | Signal | Min | Max | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Analogue inputs |  |  |  |  |  |  |  |
| FS 1 | PTO force | Futek | LSB200 | 660399 | mV/V | -50 | +50 | Lb |
| FS 2 | PTO force | Futek | LSB200 | 660406 | mV/V | -50 | +50 | Lb |
| IS 1 | Silo inclinometer | Level Developments | DAS-30-A | 46214 | 4-20mA | -30 | +30 | Deg |
| LS 2 | Silo gap level | AWS | Cap probe | AWS | +/-10V | 0 | +250 | mm |
| LS 1 | Reservoir level | IFM | LR3000 | 8 | 4-20mA | 0 | 600 | mm |
| PS 1 | Silo-reservoir diff. pressure | Impress | DMD341 | 1515110 | 4-20mA | -10 | +10 | mBar |
| PS 2 | Air supply pressure |  |  |  | 4-20mA | 0 | 5 | Bar |
| PS3 | Air delivery hose | IFM | PI 1698 |  | 4-20mA | -200 | 200 | mBar |
| LMP 1 | Linear motor position | LinMot | B1100-GP |  | A/B Enc | n/a | n/a | count |
| LE1 | Linear position encoder | Renishaw | LM150 |  | A/B Enc | n/a | n/a | count |

The following points should be noted in relation to the interface with Carderock systems:

- Position feedback is provided by way of a dedicated 4 quadrature linear encoder (Ref LE1 above) giving an A/B signal at 0.125 mm resolution.  Carderock shall provide the de-coding hardware / software.  The installation for the LM15 has been provided to Carderock separately.  The Waveswing control system will provide a digital output contact to Carderock which shall close when the system is ‘homed’ to allow setting of the zero position.

- Force feedback is provided by way of a dedicated load cell (Ref FS2).  mV/V response is provided in the calibration certificate included in Appendix [HOLD] part of which is reproduced in Figure 16 below.  Normal operation is expected to be in tension with the mean input being in the region of 40N due to spring and PTO force offsets.

Figure 16:  Kinematic sensor load cell calibration in tension

- The 4-20 mA output from the reservoir level sensor (Ref LS 1) shall be looped through the Carderock data acquisition system.  Carderock shall be responsible for provision of a signal isolation mA/mA (or mA to V) converter if required.

The interface wiring is shown on drawing 15-010 E03 included in Appendix [HOLD]

Comments:

- Force (power kinematic) (and hence end-stop impact) is measured via the PTO force transducer;

- Velocity (power dynamic) is measured via the separate linear encoder (LE 1);

- Optical tracking is underwater using small LED target mounted on the lower part of the silo;

## Motion tracking

A Natural Point tracking system will be used to track the motion of the floater using a single point measurement mounted to the bottom part of the silo.

# Data Processing and Analysis

## Data quality assurance and on-site processing

Data collection will start 2 minutes before waves are started and continue for at least 2 minutes once wave generation stops. This will ensure that the data captures the initial conditions and ramp-up/down.

“Raw” data from the Natural Point motion tracking and from the National Instruments (NI) measured power/loads/other are collected on two different systems and stored in separate text files. The motion tracking data are stored in a CSV file while the data from NI DAS are stored in a tab delimited text file.

## Data analysis

The data processing and analysis is divided into two parts: 1) data quality assurance (QA) that will ensure that quality, consistent and error free data are used in data analyses and 2) data analysis to calculate the performance metrics used in judging.  The data flow and processing steps are shown in Figure 17 and the calculated values for the data analysis are provided in Appendix J.

Post Test Analysis

Wave Sensors

Motion Tracking System

Mooring Loads Sensors

WEC PTO Sensors

Carderock DAS

Write to optical Disc

Signal Conditioning

Unit Conversion

Data Formatting

DAS Real Time Data Display

Test-by-Test Data Analysis

Signal  Conditioning and Unit Conversion

Initial QA checks

Initial Processing

Secondary QA checks

Test Report

Contestant Controller

Processing and analysis

DA Test Data Display

Figure 17. Data flow and processing steps

The objective of the data quality check is to detect and eliminate as many significant errors from the data as soon as possible, and to come to an overall assessment of the data quality.  The data QA shall be performed at three points during testing: 1) visually in “real time” during each test while data are collected, 2) during the interval after testing when the wave basin in settling and 3) when data are analyzed. It is critical to identify any data issues as soon as possible so corrective action can be taken and a test rerun if necessary.

The DAs and Carderock will decide if a test needs to be rerun if they have determined the data is of sufficiently poor quality in terms of:

- The wave field did not sufficiently match the specified spectrum

- There were errors in the measurements due to such issues as sensor failure, connector failure, too high noise, etc.

- Failure or issues with the WEC

- Fault with DAS

### “Real Time” Data QA

To ensure data quality, to prevent re-running multiple tests, and to halt tests early, all channels shall be visually monitored during testing to provide a basic level of data quality assurance and to verify that all instruments and the data acquisition system are functioning properly. If bad data are detected, the test lead should be immediately notified, who will then decide what action needs to be taken. During each test the following QA should be performed:

- Operation and performance of the DAS should be monitored to verify that it has not locked up or faulted – make sure the DAS runs throughout the test by monitoring CPU load and data updates

- Visual inspection of the data being displayed by the DAS, as they are gathered – the Carderock DAS will plot specific incoming data channels as they are acquired.

### Settling Interval and Time between Test Data QA

After each test, while the basin settles and while the next test is set up (~20 min total), a more detailed data QA shall be performed to identify any issues before the next test starts. The DA will do their best to perform this task between runs, but if this is not possible, the QA will be completed during the subsequent run. If issues are detected with the data, these will be brought up to the test lead. The following tasks should be performed:

- Operation and performance of the DAS should be monitored to verify that it has not locked up or faulted

- Time series for each data channels should be plotted and inspected

- Data shall be processed to perform higher-level data QA

- Spectra should be calculated for waves, power and loads and plotted and inspected

- Wave spectra should be compared with baseline wave spectra

- Periodic comparisons with baseline runs (as possible)

- Visual inspection of all wires, connectors and sensors should be performed (as possible)

- Visual inspection of device should be performed (as possible)

The first six bullets will be performed using pre-written scripts that interface with the Carderock DAS storage. These scripts will load the data, perform some processing, create figures for review, and identify any data of concern.

|  |  | Real Time via observation | Settling Interval | Post Test |
| --- | --- | --- | --- | --- |
| DAS malfunction | Check for data acquisition failure or malfunctions | X | X |  |
| Sensor malfunction | Check for sensor failure or malfunctions | X | X |  |
| Time difference | Check the time difference between each measurement for consistency and against specifications and check for strange variations in time |  | X | X |
| Error values/substitutes | Identify error values/substitutes (i.e. “999” or “NaN”) | X | X | X |
| Constant value | Repetitions of consecutive data with the same value (repeating standard deviations or offset). |  | X | X |
| Completeness | Check whether the number of records and their sequence is correction (identification of gaps, check for repetition) |  |  | X |
| Range /threshold | Check whether the data of each sensor lie within the measurement range of that sensor. |  | X | X |
| Measurement continuity | Compare the rate of change of a signal to expected/seasonally accepted values and between similar measurements that are collocated in close proximity |  | X | X |
| Measurement Consistency | comparison between statistics, such as the ratio of wave height to power |  |  | X |
| Near-by Comparison | Comparison with similar/duplicate measurements that are collocated in close proximity |  |  | X |
| Spectral spikes | Spikes in the spectral data |  | X | X |
| Trends and inconsistencies | Identify trends in data such as large drift in sensor output or inconstancies in sensor output for similar input |  |  | X |

# Data Management

Data are transferred from the Carderock systems (Natural Point and the NI DAS) to the DA computers via an optical media, likely a re-writable DVD or Bluray. Each disc will be labeled with the data, the team name and the included runs. Separate discs will be used for each team.

Data will be transferred to the DA computer and stored in separate directories for each team. As data are processed, the processed data, along with the processing algorithms will be stored on the DA computers. The “raw” data files SHALL not be altered by the DAs – if modifications are needed, a new file shall be created to do this, thus, preserving the original “raw” data file.

At lunch and at the end of each day, all data will be backed up to two separate hard drives and to the spare DA computer. One drive will remain at Carderock and the other will be stored at a separate location during evenings and weekends.

When DAs are using computers other than the DA computers, all analysis and algorithms will be backed up to two different jump drives at least once a day. Once analysis is complete, the DA will send one drive to the lead DA who will archive the raw and processed data on NREL’s secure data server.

The teams will not be provided with any data from Carderock or the DAs – the PAT will facilitate data transfer to the teams.

The discs, DA computer, the redundant back-up drives, and storage on NREL’s secure server provide a high level of storage redundancy

# References

[1] Prize Administration Team (2015) Wave Energy Prize Rules 5.26.15 R1.

[2] W.F. Brownell (1962) Two New Hydromechanics Facilities at the David Taylor Model Basin, Report 1690, Presented at The SNAME Chesapeake Section Meeting, December.

# Appendix A: Device electrical and mechanical drawings

# Appendix B: PTO calibration results

No Calibration performed

# Appendix c: Device Froude scaling

| Quantity | Froude Scaling | Reynolds Scaling |
| --- | --- | --- |
| wave height and length wave period and time wave frequency power density | s s0.5 s-0.5 s2.5 | s s2 s-2 s-2 |
| linear displacement angular displacement | s 1 | s 1 |
| linear velocity angular velocity | s0.5 s-0.5 | s-1 s-2 |
| linear acceleration angular acceleration | 1 s-1 | s-3 s-4 |
| mass force torque pressure power | s3 s3 s4 s s3.5 | s3 1 s s-2 s-1 |
| linear stiffness angular stiffness | s2 s4 |  |
| linear damping angular damping | s2.5 s4.5 |  |

Froude scaling R(Load), where lower case r is model scale, and capital R is full scale:

Dynamic (force) to kinematic (velocity):

rR=fFVv=s3∙s-0.5=s2.5

Dynamic (torque) to kinematic (angular velocity):

rR=τΤΩω=s4∙s0.5=s4.5

Dynamic (pressure) to kinematic (volumetric flow):

rR=pPQq=pPVvL2l2=s∙s-0.5∙s-2=s-1.5

# Provisions for dealing with air-compressibility issue s and spring effects at 1:20 scale

# (refer to AWS File Note FN 16-001)

## Operation of the air-springs within the Archimedes Waveswing

The PAT has raised the question of the dynamic similarity of the spring-like effect of air volumes within the device which cannot be Froude scaled at 1:20 scale.

Waveswing America recognize this issue and have previously addressed this successfully with the 1:50 scale tests.  The approach taken is set out in AWS file note FN 15-004 which was provided to and accepted by the PAT in August 2015.

As noted in 15-004, the Waveswing contains two pneumatic chambers which work in conjunction to produce the overall spring response of the device.  By varying the internal geometry of the device a wide range of different spring characteristics can be achieved without changing the external shape and hydrodynamics.  Use of a direct drive PTO enables further spring modification to suit conditions on a wave-by-wave basis.  The spring is therefore easily tune-able to a wide range of conditions.

Accordingly, the important consideration for model-scale is to produce an appropriate spring which produces an equivalent scaled response to that intended for the full-scale device.

As described in 15-004, it is not practicable to reproduce the vacuum (or low-pressure) air-spring chamber at model scale due to the large volume required to provide a scaled response and due to the difficulty in engineering a suitable rolling seal at small scale.

Hence, the approach adopted is to replicate the scaled spring response by using a combination of the air spring with suitable additional volume and a mechanical spring.  Again this is described in 15-004.

## Full-scale spring response (per 15-004)

The full-scale sprig characteristics for the device are described in 15-004 and Figure 2 thereof is reproduced below:

Figure 18 - Full-scale device desired spring response (after 15-004 Figure 2)

## Model-scale spring response for 1:20 model

Noting the values set out in Table 1 of 15-004, and adjusting the scale factor to 1:20, the required restoring force at model scale is set out in Table 3 overleaf.  Again repeating the comments regarding linearization of the spring during detailed design, it is considered appropriate to select a mid-range value of +/- 7.7N at the stroke ends.

Selecting a mechanical spring constant of 40 N/m and an additional air reservoir volume of 230 litre produces the required response as illustrated in Figure 19 (yellow line is actual spring curve for the model, purple line is the scaled spring curve from the full-scale design).  From this it will be seen that a good match is achieved for the majority of the stroke.  The 1:20 model will be slightly stiffer on the up-stroke and slightly less stiff on the down-stroke.  These effects are expected to cancel out in performance terms.

It should be noted that this is for one particular tuning only.  As with the full-scale device, the air volumes will be varied from sea-state to sea-state in order to produce optimal response.

| Floater position (m) | -2 | 0 | 2 |
| --- | --- | --- | --- |
| Vacuum Force (kN) | -520 | -482 | -435 |
| Air-spring force (kN) | 643 | 531 | 435 |
| Mass force (kN) | -49.1 | -49.1 | -49.1 |
| Net force (kN) | 73.9 | 0 | -48.7 |
| Position at model scale (mm) | -100 | 0 | 100 |
| Force at 1:20 scale (N) | 9.2375 | 0 | -6.0875 |

Table 9:  Spring forces at full scale and 1:20 scale

Figure 19:  Comparison between scaled spring curve and actual 1:20 model spring curve

## Effect of additional floater mass at model scale

A key feature of the Waveswing at full-scale is the low floater mass which enables a broad-banded response.  The full-scale mass of the floater and moving PTO parts is estimated at around 10 ton.  At 1:20 this scales to 1.25kg.  Unfortunately it has not been possible to achieve such a low mass at model scale and the final mass of the floater and PTO assembly is in the region of 2kg.  This requires that we increase the spring constant in order to maintain the required scaled peak frequency response.  This will be done by reducing the working volume of the reservoir.  It is however expected that the bandwidth of the device will be compromised somewhat by the additional mass at model scale.  Unfortunately, this is unavoidable and is accepted by the Waveswing America team.

## Conclusion

From the foregoing it will be seen that it is possible to produce an appropriate spring response for the 1:20 scale Waveswing device by using a combination of an additional air volume eternal to the device, and a mechanical spring.

The actual spring rate can be adjusted by varying the working volume of the external reservoir, thus providing the ability to compensate somewhat for the unavoidable additional floater mass.

The implementation of the spring at model scale does not affect the external geometry or hydrodynamics of the device, nor does it affect the kinematic or dynamic power measurements.

# Appendix D: Detailed description of control strategy

Refer to AWS document Waveswing 1:20 Scale Model Controls Description, AWS ref 16-006

# Appendix E: Raw Data Channel list

| Instructions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data File: |  | The name of the data file where the data resides |  |  |  |  |  |
| Channel Name: |  | The name of the data channel in the data file - if data are in a matrix, this is the index (column number) of the data in the matrix |  |  |  |  |  |
| Channel Title: |  | The common named use to refer to the data in the cannel |  |  |  |  |  |
| Description: |  | Description of what is being measured/recorded |  |  |  |  |  |
| Unit: |  | The unit of the measurement as output by the DAS |  |  |  |  |  |
| Sensor |  | The name of the sensor. Please provide enough information so a reader can identify the specific type of sensor used |  |  |  |  |  |
| Sample Rate |  | The sample rate of the data record |  |  |  |  |  |
| Scaling and Conversion Calculations |  | Where the measured values will be used |  |  |  |  |  |
| Data File | Channel Name | Channel Title | Description | Unit | Sensor | Sample Rate | Scaling and Conversion Calculations |
| WEC Prize Test Sensors |  |  |  |  |  |  |  |
| See Table 8 |  |  |  |  |  |  | See data file for conversions used during testing |
| Team Sensors |  |  |  |  |  |  |  |

# Appendix F: Checklists for spot checks, software operation, readiness verification, and “real time” data QA

## Spot Check Procedure

Prior to testing, the first spot check was be performed in the Carderock parking lot outside of the MASK facility. The spot check will be performed to verify the accuracy and the scale of the dynamic and kinematic sides of power which use linear encoder and the load cell respectively:

The following two spot checks were be performed:

- Load cell performance by incrementally increasing and decreasing weight added to the float. Data were collected for an interval of 60 seconds for each individual load. At least 11 increments were collected spanning the full range of the load cell. This was repeated up and down.

- Linear encoder performance covering the full range of motion. 9 increments were needed spanning the full range of the motion. The value of the linear encoder will also be recorded at the end stops.

| Spot Check 1 – Load cell |  |
| --- | --- |
| Calibration weight applied | Load cell reading |

| Spot Check 2 – Displacement |  |
| --- | --- |
| Micrometer position | Instrument reading |

For every run, the following check will be performed to ensure the WEC is ready for testing

- Check for loose bolts, appendages, and connectors

- Water level of device and float

- Verify integrity of optical markers

- Verify telemetry (loads, position, motion tracking) is recording

- Verify umbilical is free of tangles and twists

# Appendix G: Team provided sensors – specifications and calibrations

# Appendix H: Data analysis details

The following time series will be plotted for each run and be available for viewing between runs  (if time permits)

| Variable | Definition | Reference Formula | Relevant Runs |
| --- | --- | --- | --- |
| Displacement – Inertial Frame | X is defined relative to the 0 deg wave heading, Z is upward and Y completes the right hand rule | X= X,Y | All |
| Mooring Tension for each line | The instantaneous value of the mooring tension for line j | Fj | All |
| Kinematic Power | Kinematic Side of Power for PTO j | kinj | All |
| Dynamic Power | Dynamic Side of Power for PTO j | dynj | All |
| Absorbed Power | Absorbed power for PTO j | Pj | All |

The following variables will be calculated for each run and be available for viewing between runs (if time permits)

| Variable | Definition | Reference Formula | Relevant Runs |
| --- | --- | --- | --- |
| Wave PSD | Spectral density of the water surface elevation | SHf | All |
| Significant Wave Height | Measured significant wave height | Hm0=4m0 where mi=k=f0fNfkiSkf∆fk | All |
| Omni-Directional Wave Energy Flux | Omni-Directional Wave Energy Flux | J= ρgk=f0fNSkfCgk∆fk Cgk= 12Cpk1+2kkhsinh2kkh Cpk= gkktanhkkh | All |
| Wave Energy Period | Wave Energy Period | Te=m-1m0 | All |
| Horizontal Displacement | Horizontal displacement of the WEC from its at rest position | Υ=X2+Y2 | All |
| Mean | The mean value of the mooring tension for line j | Fj=1Ni=1NFji | All |
| Standard Deviation | The standard deviation of the mooring tension for each mooring line | σFq=1N-1i=1MFji-Fj2 | All |
| Max | The maximum value of the mooring tension of all mooring lines | maxFj | All |
| Min | The minimum value of the mooring of all mooring lines | minFj | All |
| Mean | The mean value of the kinematic side of power for PTO j | kinj=1Ni=1Nkinji | All |
| Standard Deviation | The standard deviation of the kinematic side of power for PTO j | σkinj=1N-1i=1Mkinji-kinj2 | All |
| Max | The maximum value of the kinematic side of power | maxkinj | All |
| Min | The minimum of the kinematic side of power for PTO j | minkinj | All |
| Kinematic spectral density | Spectral density of the kinematic side of power for PTO j | Skinjf | All |
| Mean | The mean value of the dynamic side of power for PTO j | dynj=1Ni=1Ndynji | All |
| Standard Deviation | The standard deviation of the dynamic side of power | σdynj=1N-1i=1Mdynji-dynj2 | All |
| Max | The maximum value of the dynamic side of power | maxdynj | All |
| Min | The minimum of the dynamic side of power | mindynj | All |
| dynamic spectral density | spectral density of the dynamic side of power, one for each power conversion chain of the WEC | Sdynjf | All |
| Mean | The mean value of the power | Pj=1Ni=1NPji | All |
| Standard Deviation | The standard deviation of the power | σPj=1N-1i=1MPji-Pj2 | All |
| Max | The maximum value of the power | maxPj | All |
| Min | The minimum of the power | minPj | All |
| Absorbed power spectral density | spectral density of the absorbed power, one for each power conversion chain of the WEC | SPjf | All |

This page intentionally left blank