421010-R, Rev. 00

Report for the CFD Analysis of the 
Self-Rectifying, Mono-Radial Impulse Turbine

Prepared for:

Hanna Wave Energy Primary Drives

March 2022

| Name/Title | Signature/Date | Preparer (P) Reviewer (R) |
| --- | --- | --- |
| Duncan Phyfe Senior Engineer |  | P |
| David C. White Director, Gas Flow Engineering |  | R |

Revision History

| Rev. No. | Date | Change Description |
| --- | --- | --- |
| 00 | 3/9/2022 | Original issue |

Contents

1.0	Introduction	6

2.0	Device Description	7

3.0	Literature Review	8

3.1	Literature Findings	8

3.2	Literature Modeling Inputs	8

4.0	Deviation from the Test Plan	9

5.0	Analysis Methods	10

5.1	CFD Model Definition	11

5.2	2D CFD Analysis	16

5.3	3D CFD Analysis	18

6.0	Project Outcomes	21

6.1	Results	21

6.2	Lessons Learned	39

7.0	Conclusions and Recommendations	43

8.0	References	44

9.0	Acknowledgements	45

List of Figures

Figure 5-1: Model Mesh Overview: (left) 2D Hanna Turbine; (right) 3D 8-Nozzle	11

Figure 5-2: Model Inlet Mesh Detailed View: (left) 2D Hanna Turbine; (right) 3D 8-Nozzle	11

Figure 5-3: 2D Models: (left) Representative Hanna Turbine; (right) Turbine Blade Angle 2 (55°)	12

Figure 5-4: 2D & 3D Models: (left) 2D Representative Hanna Turbine; (right) 3D Turbine Blades (1”)	13

Figure 5-5: 2D & 3D Models: (left) Blade Angle 2 (55°); (right) 3D 2-Nozzle Blade Angle 2 (55°)	13

Figure 5-6: 3D Models: (left) 3D 2-Nozzle Blade Angle 2 (55°); (right) 3D 8-Nozzle Blade Angle 2 (55°)	14

Figure 5-7: Test Plan 8-Nozzle Conceptual Sketch and 3D Geometry	15

Figure 5-8: 8 Nozzle Test Plan Concepts (left) Staggered Nozzles; (right) Inline Nozzles	16

Figure 5-9: 2D CFD Model Idealized Flapper Valve Arrangement	17

Figure 5-10: 3D Model, Flapper Valve Arrangement	19

Figure 6-1: Overview Torque for the Flow Expansion 2D CFD Cases	21

Figure 6-2: Summary Plot of the Top 3 Performing 2D Turbine models	23

Figure 6-3: 2D Analysis Results Highlighting the Impact of Model Inlet Pressure	24

Figure 6-4: 3D Model Results	25

Figure 6-5: Contours of Pressure (left) 2D Hanna Turbine; (right) 2D Blade Angle 2	26

Figure 6-6: Contours of Pressure (left) 2D Hanna Turbine; (right) 3D Blade A	26

Figure 6-7: Contours of Pressure (left) 2D Blade Angle 2; (right) 3D 2-Nozzle Blade Angle 2	27

Figure 6-8: Contours of Pressure (left) 3D 2-Noz. Blade Angle 2; (right) 3D 8-Noz.Blade Angle 2	27

Figure 6-9: Contours of Velocity (left) 2D Hanna Turbine; (right) 2D Blade Angle 2	28

Figure 6-10: Contours of Velocity (left) 2D Hanna Turbine; (right) 3D Blade 2	29

Figure 6-11: Contours of Velocity (left) 2D Blade Angle 2; (right) 3D 2-Nozzle Blade Angle 2	29

Figure 6-12: Contours of Velocity (left) 2D 2-Noz. Blade Angle 2; (right) 3D 8-Noz. Blade Angle 2	30

Figure 6-13: Contours & Vectors, Model Inlet, (left) 2D Hanna Turbine; (right) 3D Blade Angle 2	30

Figure 6-14: Velocity Vectors, Model Inlet, (left) 2D Hanna Turbine; (right) 3D Short Blade Angle 2	31

Figure 6-15: Velocity Vectors, Model Outlet, (left) 2D Hanna Turbine; (right) 3D Short Blade Angle 2	32

Figure 6-16: Velocity Vectors (left) 3D 2-Nozzle; (right) 3D 8-Nozzle	33

Figure 6-17: Velocity Vectors 3D 8-Nozzle Turbine	34

Figure 6-18: Velocity Vectors Flow Inlet & Outlet, 3D 8-Nozzle Turbine	35

Figure 6-19: Contours of Pressure (left) Staggered Nozzles; (right) Inline Nozzles	36

Figure 6-20: Contours of Velocity (left) Staggered Nozzles; (right) Inline Nozzles	36

Figure 6-21: Velocity Contours, Mechanically Rectified Unidirectional Turbine Rotation	37

Figure 6-22: Velocity Vectors, Mechanically Rectified Unidirectional Turbine Rotation	37

Figure 6-23: 2D Model Idealized Flapper Valve, Inlet/Outlet Flow Overview (1)	38

Figure 6-24: 2D Model Idealized Flapper Valve, Inlet/Outlet Flow Overview (2)	38

Figure 6-25: 2D Models: (left) Simplified Radial & (right) Hanna Turbine, Mesh Shown	39

Figure 6-26: 2D Models, Torque Comparison: Hanna Turbine vs. Simplified Radial Turbine	40

Figure 6-27: 2D Models: Velocity Vectors (left) Simplified Radial & (right) Hanna Turbine	41

Figure 6-28: 2D Models: Velocity Contours (left) Simplified Radial & (right) Hanna Turbine	41

Figure 6-29: 2D Models: Pressure Contours (left) Simplified Radial & (right) Hanna Turbine	42

List of Tables

Table 5-1: CFD Model Summary	12

Table 5-2: 2D CFD Model Inputs	18

Table 5-3: 3D CFD Model Inputs	20

Table 6-1: 2D Results from the Representative Hanna Turbine (DP = Design Point)	22

Table 6-2: 2D Results from the Blade A  Turbine	22

Table 6-3: 2D Results from the Blade B Turbine	23

Table 6-4: 3D Model Results	24

Table 6-5: 2D Simple Radial Turbine Results	40

# Introduction

The following report is a predictive numerical and Computational Fluid Dynamics (CFD) study undertaken by Alden Research Laboratory of Massachusetts; U.S.A. Alden researchers were tasked to evaluate the Hanna Mono-radial turbine in its early design phase. The study was funded by the U.S. Department of Energy through its TEAMER (Testing and Expertise for Marine Energy) Program [1].  The turbine was invented by John Clark Hanna of Oregon, U.S.A. [2].

The details of the report are intended to be “Open Source” materials which shall exist in the public domain. It is the inventor’s desire to make this information available to all developers around the world to stimulate competition within the nascent marine renewable energy sector. This will foster improvements of the design and help lower the cost of marine energy making it more affordable in the marketplace.

The modeling effort focuses on the evaluation of a small 12-inch diameter air turbine used as a wave energy converter (WEC). A generator is driven by alternating air flows created by the up and down motion of waves. A simple arrangement of flow-activated, V-shaped “flapper hinges,” rectifies the bi-directional air streams to allow the turbine’s rotor to spin in one direction only. The study compared various design iterations of two or eight inlet nozzle configurations. Computational modeling served to predict the turbine’s performance metrics. The data showed how the device performs as an air-driven wave energy power take-off (PTO).

# Device Description

The device is an impulse-type mono-radial air turbine PTO for wave energy conversion. The turbine is self-rectified, meaning that it spins in one direction only while capturing the bi-directional air flows developed within an OWC (Oscillating Water Column) system. The turbine’s rotor resembles a water wheel that is driven by air. Reciprocating air flows are developed by two prime mover options:

- One prime mover is the rise and fall of the air/water interface within an OWC system. This prime mover is typical of an OWC arrangement and is known as an ‘open-ended’ system, where it pushes compressed air through the turbine and exhausts into the atmosphere as water levels rise inside the OWC wave capture chamber. When the water level recedes, air is sucked back through the turbine and into the wave chamber, completing one full wave cycle. When measuring the back-and-forth air movements within a typical OWC, the flows are seen to be uneven; air that is pushed out of the system is more dynamic than the air which is sucked back in. It will also power floating OWC-type buoys

- The second prime mover develops a balanced flow of reciprocating air. It is created within a ‘closed loop’. A sealed system such as this can be energized by the action of opposing sets of bellows or bladders. When one set of bellows are collapsing, the opposing set expands at the same time creating a full, free-flowing action upon the turbine’s rotor. This system will provide low-wattage power for charging batteries on small data collection and research buoys [2].

The air is injected into the sealed turbine through two nozzles. The reciprocating flows are mechanically rectified to cause the turbine’s rotor to spin in one direction only. The one-way rotary motion is achieved by simple, flow-activated, V-shaped ‘flapper hinges’ that redirect the incoming and outgoing flows to give the turbine its one-way driving force. The device has two air injection options to be evaluated: a two-nozzle delivery system and an eight-nozzle configuration.

# Literature Review

Literature sources were reviewed for applicable and existing design standards.

## Literature Findings

The literature research showed that the top OWC turbines are:

- Wells turbine

- Radial impulse

- Bi-radial

Based on the review there were no other documented impulse-type mono-radial air turbine PTO matching the Hanna Wave Energy approach. The lack of literature available for similar turbines showed that the approach for extracting the wave energy was novel.

The major turbine flow characteristic that separates the Hanna Wave Energy turbine was the circumferential flow path around the turbine. The turbine is operated via an inlet and outlet located on the outside diameter of the device.

The impulse-type mono-radial air turbine PTO is closer to the Pelton Turbine than a radial turbine.

## Literature Modeling Inputs

Following the conclusions of the turbine type, attention was directed to documenting the available energy input for the modeling efforts. The review showed a reasonable starting point for the turbine analysis to focus on a pressure input that was between 5-15 kPa.

# Deviation from the Test Plan

The use of BEM was tabled due to the flow configuration through the mono-radial turbine. The flow characteristics around the outside of the blade verse through the blade invalidates the use of BEM. Therefore, 2D CFD was used to evaluate the initial turbine performance.

The 2D flow characteristics in the mono-radial turbine are well suited to starting with a 2D model. The initial 2D modeling efforts would focus on an idealized turbine geometry to estimate the performance of the device. Based on the finding of the 2D modeling the 3D model would be developed to provide a more accurate view of the turbine performance.

The Flow3D model to evaluate the air-to-water interface was eliminated; based on the lower than desired device performance. The 2D modeling effort was expanded to capture further geometric features and design variables. The inventor intends to continue working on this device and will need to address the air-water interface once he is successful at increasing the performance.

The current turbine design torque performance was found to be lower than desired, and not sufficient for most use applications. Therefore, further testing, specifically the transient flapper valve analysis, was shelved to better understand the root cause of low power performance.

A reference model was determined to help show the level of expected and desired performance. The reference model was a simplified radial turbine sized like the Hanna turbine. This modeling effort would remain in the 2D space.

# Analysis Methods

The original plan to use a BEM model was changed after a detailed review of the turbine concept. Instead, an idealized 2D model of the turbine was developed. The idealized model reduces the complexities of the original device with the following:

- Mock flapper valve using walls to redirect the flow

- Provide tangential inlet nozzle

- More formal diffuser at the exit of the centrifugal model outlet

The ANSYS-Fluent commercial CFD package was used for all CFD modeling performed.

The CFD model physics:

- 2D/3D Steady-State

- Iso-thermal

- 2-equation turbulence model: SST k-Omega

- 2nd order discretization for all solution variables

- Turbo machinery is model using the moving reference frame model

- Boundary conditions:

  - Inlet: pressure

  - Outlet: specified pressure

  - Wall: hydraulicly smooth non-slip

The mesh used unstructured hexahedral (2D) and hybrid polyhedral (3D) meshes. The resolution was fit to capture the gradients within the flow. Figure 5-1 shows the mesh used in the 2D and 3D models.

Figure 5-1: Model Mesh Overview: (left) 2D Hanna Turbine; (right) 3D 8-Nozzle

Figure 5-2 provides a detailed view of the model inlet zone.

Figure 5-2: Model Inlet Mesh Detailed View: (left) 2D Hanna Turbine; (right) 3D 8-Nozzle

## CFD Model Definition

Table 5-1 provides a summary of the CFD model geometries evaluated. These variations allow for understanding the performance impact based on geometric changes.

Table 5-1: CFD Model Summary

A comparison of various models is shown in Figure 5-3 through Figure 5-6. These models are a combination of 2D and 3D models. The inlet and outlet boundaries are highlighted on each figure. For example, Figure 5-3 shows the inlets to both 2D models in the lower right and the outlet at the top left.

Figure 5-3: 2D Models: (left) Representative Hanna Turbine; (right) Turbine Blade Angle 2 (55°)

Figure 5-4 shows the 2D model on the left and 3D model on the right, again with the pressure boundary conditions highlighted.

Figure 5-4: 2D & 3D Models: (left) 2D Representative Hanna Turbine; (right) 3D Turbine Blades (1”)

Figure 5-5 shows a comparison view of the 2D and 3D models. The 3D model was updated to have a 360° inlet to provide a more consistent comparison to the 2-nozzle and 8-nozzle systems.

Figure 5-5: 2D & 3D Models: (left) Blade Angle 2 (55°); (right) 3D 2-Nozzle Blade Angle 2 (55°)

Figure 5-6 shows the model comparison between the 2-nozzle and 8-nozzle arrangements. These models are both 3D. The 8-nozzle design was only considered in 3D.

Figure 5-6: 3D Models: (left) 3D 2-Nozzle Blade Angle 2 (55°); (right) 3D 8-Nozzle Blade Angle 2 (55°)

The test plan included a view of the 8-nozzle arrangement (Figure 5-7). This arrangement did not use the flapper valve. The inventor envisioned the turbine functioning like a De Laval impulse turbine. The staggered inlet arrangement shown in Figure 5-7 created concerns around the complexity of the flow paths exiting the turbine and possible blade force maldistribution.

Figure 5-7: Test Plan 8-Nozzle Conceptual Sketch and 3D Geometry

Two 3D CFD models were created for this 8-nozzle arrangement without the flapper valve: the first with staggered nozzles and the second with the nozzles inline. Figure 5-8 presents the two CFD models.

Figure 5-8: 8 Nozzle Test Plan Concepts (left) Staggered Nozzles; (right) Inline Nozzles

## 2D CFD Analysis

2D analyses were used to quickly investigate turbine performance under various boundary and geometric conditions. These models run much quicker than 3D simulations. The 2D models utilize a simplified flapper valve that  provided an idealized tangential inlet nozzle and outlet diffuser for the turbine (Figure 5-9). The inlet nozzle provides an accelerated flow into the device and the outlet diffuser improves the flow expansion exiting the device.

The flow redirection of the flapper valve was achieved with walls.

Figure 5-9: 2D CFD Model Idealized Flapper Valve Arrangement

The inputs for the 2D analysis are shown in Table 5-2. A Design of Experiments (DoE) approach was used to supply the CFD models with a table of inputs to simulate and report performance values. This DoE style of analysis can be key in the determination of an optimized design. The DoE technique was used to provide an analysis of the torque for each model configuration at a wide range of pressure inputs and rotor speeds.

Table 5-2: 2D CFD Model Inputs

## 3D CFD Analysis

The 3D model shown in Figure 5-10 highlights the inlet and outlet flapper valve flow redirection.

Figure 5-10: 3D Model, Flapper Valve Arrangement

The addition of the flapper valve created a flow obstruction that provided substantial impact to the entering flow. This was a dramatic change from the 2D analysis and lower torque values were expected as a result.

Table 5-3: 3D CFD Model Inputs

# Project Outcomes

## Results

The torque is predicted through the forces and moments on the rotor. The sign (positive or negative) of the torque is the expansion or compression of the air. The positive torque values calculated represent an expansion of the air. The expanding air through a turbine is the reason for the typical inlet nozzle and outlet diffuser; however, this application requires a more neutral geometry as the turbine spins only in a single direction no matter the flow direction.

The 2D results are presented in Figure 6-1. Only the model cases producing a positive torque value are reported.

Figure 6-1: Overview Torque for the Flow Expansion 2D CFD Cases

Tabulated in Table 6-1 through Table 6-3 the top 3 performing cases from the 2D are summarized. These cases were used to decide what cases were analyzed in 3D.

Table 6-1: 2D Results from the Representative Hanna Turbine (DP = Design Point)

Table 6-2: 2D Results from the Blade A  Turbine

Table 6-3: 2D Results from the Blade B Turbine

The tabulated results can be seen in Figure 6-2 plotted for a visual summary. All three cases are similar.

Figure 6-2: Summary Plot of the Top 3 Performing 2D Turbine models

The Hanna turbine 2D results are presented in Figure 6-3 showing the torque impact as function of model inlet pressure vs. rotor speed. The literature review provided a basis for the selection of inlet pressure range 5-15 kPa, the model runs included pressures down to 250 Pa.

Figure 6-3: 2D Analysis Results Highlighting the Impact of Model Inlet Pressure

The tabulated results for the 3D model runs are shown in Table 6-4.

Table 6-4: 3D Model Results

The 3D model results are presented graphically in Figure 6-4. Positive values of torque are the goal.

Figure 6-4: 3D Model Results

Using the same layout of the earlier figures, the following will walk through a graphical comparison of the various models. The data is taken from the top 2D model runs (Figure 6-2) and the 3D model runs (Figure 6-4). The inlet pressure (10 kPa) and the rotation (1000 rpm) were the same for all figures.

Figure 6-5 and Figure 6-6 show the pressure profiles that are similar through the models. The 3D shows the adverse impact of the flapper valve on local pressure profile. Note that the flapper valve is idealized in the 2D models, so the results over-predict performance.

Figure 6-5: Contours of Pressure (left) 2D Hanna Turbine; (right) 2D Blade Angle 2

Figure 6-6: Contours of Pressure (left) 2D Hanna Turbine; (right) 3D Blade A

The addition of the 360° affects the pressure profile as the flow needs to go through an inlet port to reach the turbine proper. Figure 6-7 compares the 2D and 3D cases. The 2D model predicted a positive torque, whereas the 3D model predicted a negative torque.

Figure 6-7: Contours of Pressure (left) 2D Blade Angle 2; (right) 3D 2-Nozzle Blade Angle 2

Figure 6-8 compares the 2-nozzle and 8-nozzle arrangements for the turbine. The 360° inlet was created to provide a consistent inlet comparison between the 2 nozzle and 8 nozzle models. This provides a view of how the air intake would impact the pressure profile through the device.

Figure 6-8: Contours of Pressure (left) 3D 2-Noz. Blade Angle 2; (right) 3D 8-Noz.Blade Angle 2

The velocity-based graphics highlight the limited use of the blade passageway. The circumferential flow primarily interacts with the blade tips. The alternating flow is only available to 50% of the blades at one time in the 2-nozzle arrangement. The 8-nozzle arrangement addressed this by using more inlets and outlets. The results showed an increase in overall performance for the 8-nozzle arrangement (Table 6-4).

Figure 6-9 and Figure 6-10 show the primary flow path through the two models. There are some minor differences in the velocity profiles due to the angle changes in the blades.

Figure 6-9: Contours of Velocity (left) 2D Hanna Turbine; (right) 2D Blade Angle 2

Figure 6-10: Contours of Velocity (left) 2D Hanna Turbine; (right) 3D Blade 2

There is a dramatic entrance effect seen in Figure 6-11 created by the flapper valve (3D). The flow from the flapper is not tangential entering the turbine. This creates an energy loss as the flow changes direction and reduces the overall performance of the turbine.

Figure 6-11: Contours of Velocity (left) 2D Blade Angle 2; (right) 3D 2-Nozzle Blade Angle 2

Figure 6-12 shows the common velocity profile in the 3D models. Noting the velocity profile change due to blade alignment at 10 o’clock with the 8-nozzle model.

Figure 6-12: Contours of Velocity (left) 2D 2-Noz. Blade Angle 2; (right) 3D 8-Noz. Blade Angle 2

The entrance flow is shown for comparison in Figure 6-13. The idealized flow in the 2D model sets the flow through the device up substantially more tangentially producing an improved flow profile. The velocity vectors provide flow directionality at the blades.

Figure 6-13: Contours & Vectors, Model Inlet, (left) 2D Hanna Turbine; (right) 3D Blade Angle 2

Figure 6-14 removes the underlying velocity contour from Figure 6-13 allowing the vectors to highlight the dominate flow path.

Figure 6-14: Velocity Vectors, Model Inlet, (left) 2D Hanna Turbine; (right) 3D Short Blade Angle 2

Figure 6-15 looks at the outlet flow profile with the same model runs as Figure 6-14. The 2D model outlet was enlarged to supply more flow expansion at the exit. Based on the velocity profiles through the gap, the geometric feature in the 2D model is not providing significant positive flow characteristic.

Figure 6-15: Velocity Vectors, Model Outlet, (left) 2D Hanna Turbine; (right) 3D Short Blade Angle 2

There are options to develop a flapper valve arrangement that may create an improved inlet nozzle. In the scope of this analysis, the simplified nozzle still under-performed with low torque values. Any changes to the flapper to make a substantial impact on the turbine performance would likely include changes to the flow through the blade passages and are out of the current scope of work.

In Figure 6-16 the flow characteristics between the 2-nozzle and 8-nozzle are highlighted. Neither the 2 nor 8 nozzle arrangement provided a flow expansion at 1000 rpm; however, the 8-nozzle did provide an expansion of the flow at 250 rpm. This is an improvement over the 2-nozzle arrangement.

Figure 6-16: Velocity Vectors (left) 3D 2-Nozzle; (right) 3D 8-Nozzle

The colorized vectors in Figure 6-17 and Figure 6-18 provide a view of the velocity profile through the device. The view of the 8-nozzle inlet and outlet are shown in Figure 6-18.

Figure 6-17: Velocity Vectors 3D 8-Nozzle Turbine

Figure 6-18: Velocity Vectors Flow Inlet & Outlet, 3D 8-Nozzle Turbine

The 8-nozzle arrangement from the test plan results are shown in Figure 6-19 and Figure 6-20. The tabulated results in Table 6-4 show the performance of the staggered nozzle to be on par with the 8-nozzle flapper valve arrangement. However, the performance of the inline arrangement is poor by comparison. Without the mechanical redirection of the flapper valve, the flow inefficiency (energy loss) is at the outlet connection.

Figure 6-19: Contours of Pressure (left) Staggered Nozzles; (right) Inline Nozzles

Figure 6-20: Contours of Velocity (left) Staggered Nozzles; (right) Inline Nozzles

Section 2.0 discusses the reciprocating flows are mechanically rectified to cause the turbine’s rotor to spin in one direction only. The CFD modeling efforts considered the steady-state operation in either flow direction based on a pressure differential. This could be considered an open or closed loop operation. As noted previously the closed loop would be highly dependent on the air-water interface.

The following figures (Figure 6-21 to Figure 6-24) show the turbine operating through ‘Path A’ then ‘Path B’. The first figures are velocity contours followed by velocity vector plots showing the details of the flows entering/exiting the turbine.

Figure 6-21: Velocity Contours, Mechanically Rectified Unidirectional Turbine Rotation

Figure 6-22: Velocity Vectors, Mechanically Rectified Unidirectional Turbine Rotation

Figure 6-23: 2D Model Idealized Flapper Valve, Inlet/Outlet Flow Overview (1)

Figure 6-24: 2D Model Idealized Flapper Valve, Inlet/Outlet Flow Overview (2)

## Lessons Learned

The first take-away from this work is the turbine inlet nozzle importance. It was shown in the changes in performance from the 2D to 3D turbine.

Likewise, the turbine must use the blade passage to create torque more effectively for the system.

As a reference, a simplified radial turbine was modeled in 2D to compare with the Hanna turbine model (2D), Figure 6-25. Under OWC boundaries the radial turbine provides low loss entry and exit. The flow is improved as it passes through the turbine radially outward or inward without mechanical redirection. Some radial turbines utilize flow controls to improve the turbine performance, however none were used in the reference model.

Figure 6-25: 2D Models: (left) Simplified Radial & (right) Hanna Turbine, Mesh Shown

As shown in Figure 6-26 the simplified radial turbine produced torque roughly two orders of magnitude higher than the Hanna turbine.

Figure 6-26: 2D Models, Torque Comparison: Hanna Turbine vs. Simplified Radial Turbine

Table 6-5 tabulates the model results for the simplified radial turbine.

Table 6-5: 2D Simple Radial Turbine Results

Figure 6-27 illustrates the blade engagement differences between the radial and Hanna turbines. The radial turbine engages the full blade passageway and flows through all the blades. The Hanna turbine blade interaction is with the turbine tip.

Figure 6-27: 2D Models: Velocity Vectors (left) Simplified Radial & (right) Hanna Turbine

Figure 6-28 shows the velocity profile through the two CFD models.

Figure 6-28: 2D Models: Velocity Contours (left) Simplified Radial & (right) Hanna Turbine

Figure 6-29 captures the pressure profile difference between the two turbine models.

Figure 6-29: 2D Models: Pressure Contours (left) Simplified Radial & (right) Hanna Turbine

The low torque nature of the Hanna turbine would suit itself well with a low speed and low torque motor.

The target application is to provide low-wattage battery charging power for small autonomous data collection buoys. The low performance characteristics of the device are suited for charging applications where the device would work as a trickle charging driver verse a high-powered rapid charger.

There were various generator options discussed with the bulk of the initial low torque motors being higher speed. The inventor has identified a generator that will require further discussion with the manufacture to determine if a match exists. Geometrically, both devices appear to match well. Alden has reviewed some of the supplier generator data, and it would show that it would require much higher torque than the Hanna Turbine is producing. These machines are showing 15 - 750 kNm torque [4, 5] compared to 0.01 kNm from the best performing 2D simulation. The high torque value is for a larger machine and scaling the diameter down would decrease the torque requirement. The available documentation did not provide any guidance for scaling the generator.

# Conclusions and Recommendations

In this predictive numerical study, the Hanna Mono-radial Turbine was analyzed in its initial stages of development.  It showed that it can operate as a torque producing turbine.  The torque values are lower than desired. The inventor has devised two turbine iterations which use different nozzle configurations. The data predicts that either nozzle arrangement provide similar performance. In 2D the maximum estimated torque for the 12” rotor is 10 Nm; whereas, in 3D estimated torque is less 1 Nm.

The in-house bench testing performed by the inventor, focused on visual characteristics of the device and no specific measurements were taken to quantify the level of performance through the tests. The needed measurements to quantify the performance are not typical in an inventor’s shop [2].

As discussed in Section 6.2, the initial target for the mono-radial turbine would be as a charging system for small data collection-type buoys. The lower-than-expected performance in this early-stage design would require a specific system where the generator and turbine are matched for a low torque, low rotational speed application. The specifics around the OWC were tabled in this analysis to look at an expanded range of geometric features (Section 4.0).

The mono-radial turbine performance characteristics are lower than desired:

- Lack of blade engagement as the circumferential flow bypasses the blade passageway,

- Complex flow path needed to enter the device around the flapper value, or

- Complex flow in the exit of the 8 Nozzle staggered non-flapper valve device.

Armed with the lessons learned through this project the inventor should continue his pursuit to foster improvements of the design and help lower the cost of marine energy making it more affordable in the marketplace.

# References

TEAMER Program, https://www.teamer-us.org

Hanna Wave Energy Primary Drives, Coos Bay, Oregon, Bus. Lic. No. 9098; DUNS No.78679864; https://www.wetgen.com; Mono-Radial Turbine:  https://youtu.be/E1TWdQkGeU4

Hanna Wave Energy Primary Drives, Subsurface Buoy: https://www.wetgen.com/BUOY.htm.

Mueller, M., McDonald, A., (2007). A Novel Lightweight Permanent Magnet     Generator for Direct Drive Power Take Off in Marine Renewable Energy Converters, Proc. of the European Wave & Tidal Energy Conf., Porto, Portugal, September 2007

Fountain Design Ltd., https://www.fountaindesign.co.uk

# Acknowledgements

Inventor: John Hanna, Hanna Wave Energy