TEAMER 13 Final Report, 2025-0042 data_README.docx

Data and models used in subject report.

- Folder ovrVolt_30sep2024

  - Collection of MATLAB *.mat files. This is data correlating wave state to PMG output characteristics. A description of the data is provided in the spreadsheet overVoltageEvents teamer13 30sep2024.xlsx. Generated by C-Power using Ansys AQWA NAUT v16.

- Spreadsheet overVoltageEvents teamer13 30sep2024.xlsx

  - Spreadsheet summarizing data describing the simulation output data in folder ovrVolt_30sep2024. Worksheets:

    - mat file variables – variable names and descriptions. Model description.

    - overview – Summary of model output

    - ovrVolt – description of each overvoltage event (trip threshold 7.61 RPM)

- MATLAB file: UpdatedEnergyCalculationsRpt.m

  - Matlab file that analyzes the simulation data from ovrVolt_30sep2024 MATLAB files using information from the system electrical simulation.

  - Looks at the effect of different overvoltage trip points enabled by voltage mitigation means. Baseline is 7.61 RPM. Tuning of built-in measures raises to 8.5 RPM. Implementation of flux weakening raises to 10 RPM.

  - Looks at the effect of rapidly reconnecting after a trip via a fast acting switch versus a longer off-time of a conventional circuit breaker.

  - Cases analyzed are as follows:

    - Case 1. Ideal All Energy Available

    - Case 2. Baseline (7.61 RPM) with 30 s recovery

    - Case 3. 8.5 RPM, 30s recovery

    - Case 4. 10 RPM (FW),30s recovery

    - Case 5. 10 RPM (FW), 50V recovery

- Spreadsheet: energyCalculations 26FEB26.xlsx

  - Excel file output from UpdatedEnergyCalculationsRpt.m. Output as described above.

- Spreadsheet: energyCalculations with SCR losses 26FEB26 OC 7.61, 8.5, 10.0 w C-Power trip metrics.xlsx

  - Excel file that processes energyCalculations 26FEB26.xlsx to produce Tables 1, 2, 4, and 5 in the report. energyCalculations 26FEB26.xlsx is to be copied and pasted into the sheet called Matlab Output.

  - Incorporates information from overVoltageEvents teamer13 30sep2024.xlsx on the overview tab to characterize the baseline condition.

  - Converts overall energy from energy to average power.

  - Calculates effects of SCR losses for cases with a solid-state switch.

  - Formats data in a presentable way for the report.

- Spreadsheet: CY20381-EfficiencyMapCalculation_JP2024_master.xlsm

  - Excel file from permanent magnet generator OEM presenting the relationship between PMG rotational speed, torque, mechanical power, current, losses, electrical power, voltage, and efficiency over a speed range from 0 RPM to 14 RPM.